1	RST® CLASSIC - Circular connector system IP66/68(3 m;2 h)/69 - 3-pole - RST25i3	2

1.1	Connector	2

1.1.1	Cable diameter 6-10 mm	2

1.1.1.1	Female with screw connection	2

1.1.1.2	Male with screw connection	4

1.1.2	Cable diameter 10-14 mm	5

1.1.2.1	Female with screw connection	5

1.1.2.2	Male with screw connection	6

1.1.3	Cable diameter 13-18 mm	7

1.1.3.1	Female with screw connection	7

1.1.3.2	Male with screw connection	8

1.2	Device connector	9

1.2.1	M16	9

1.2.1.1	Female with screw connection	9

1.2.1.2	Male with screw connection	10

1.2.2	M20	11

1.2.2.1	Female with screw connection	11

1.2.2.2	Male with screw connection	12

1.2.3	M25	13

1.2.3.1	Female with screw connection	13

1.2.3.2	Male with screw connection	14

1.4	Cable assemblies	15

1.4.1	Cable assemblies, 4.0 mm², fire class Eca, containing halogen	15

1.4.2	Cable assemblies, 4.0 mm², insulating rubber compound	17

2	RST® CLASSIC - Circular connector system IP66/68(3 m;2 h)/69 - 5-pole - RST25i5	19

2.1	Connector	19

2.1.1	Cable diameter 6-10 mm	19

2.1.1.1	Female with screw connection	19

2.1.1.2	Male with screw connection	20

2.1.2	Cable diameter 10-14 mm	21

2.1.2.1	Female with screw connection	21

2.1.2.2	Male with screw connection	22

2.1.3	Cable diameter 13-18 mm	23

2.1.3.1	Female with screw connection	23

2.1.3.2	Male with screw connection	24

2.2	Device connector	25

2.2.1	M16	25

2.2.1.1	Female with screw connection	25

2.2.1.2	Male with screw connection	26

2.2.2	M20	27

2.2.2.1	Female with screw connection	27

2.2.2.2	Male with screw connection	28

2.2.3	M25	29

2.2.3.1	Female with screw connection	29

2.2.3.2	Male with screw connection	31

2.4	Cable assemblies	33

2.4.1	Cable assemblies, 4.0 mm², fire class Eca, containing halogen	33

2.4.2	Cable assemblies, 4.0 mm², insulating rubber compound	35

- RST® CLASSIC - Circular connector system IP66/68(3 m;2 h)/69 - 3-pole - RST25i3

  - Connector

    - Cable diameter 6-10 mm

      - Female with screw connection

RST® circular connector, IP66/68(3 m;2 h)/69, female, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, female, 3-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Cable diameter: 6.0 - 10.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S B1 ZR2SV GR03

Part No.: 96.031.4054.3

RST® circular connector, IP66/68(3 m;2 h)/69, female, angled 90°, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, female, angled 90°, 3-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Cable diameter: 6.0 - 10.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S B1 ZR2SV GR03

Part No.: 96.033.4054.3

RST® circular connector, IP66/68(3 m;2 h)/69, splitter connector, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, female with splitter connector (two cable glands), 3-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Terminal cross-section: 0.75 - 2.5 mm² (without ferrules)

Cable diameter: 6.0 - 10.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 2

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3F B2 ZRX GR03

Part No.: 96.031.4253.3

      - Male with screw connection

RST® circular connector, IP66/68(3 m;2 h)/69, male, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, male, 3-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Cable diameter: 6.0 - 10.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S S1 ZR2 V BG03

Part No.: 96.032.4054.3

RST® circular connector, IP66/68(3 m;2 h)/69, male, angled 90°, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, male, angled 90°, 3-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Cable diameter: 6.0 - 10.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S S1 ZR2 V BG03

Part No.: 96.034.4054.3

    - Cable diameter 10-14 mm

      - Female with screw connection

RST® circular connector, IP66/68(3 m;2 h)/69, female, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, female, 3-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Cable diameter: 10.0 - 14.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S B1 ZR2SV GR03

Part No.: 96.031.4154.3

RST® circular connector, IP66/68(3 m;2 h)/69, female, angled 90°, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, female, angled 90°, 3-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Cable diameter: 10.0 - 14.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S B1 ZR2SV GR03

Part No.: 96.033.4154.3

      - Male with screw connection

RST® circular connector, IP66/68(3 m;2 h)/69, male, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, male, 3-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Cable diameter: 10.0 - 14.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S S1 ZR2 V BG03

Part No.: 96.032.4154.3

RST® circular connector, IP66/68(3 m;2 h)/69, male, angled 90°, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, male, angled 90°, 3-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Cable diameter: 10.0 - 14.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S S1 ZR2 V BG03

Part No.: 96.034.4154.3

    - Cable diameter 13-18 mm

      - Female with screw connection

RST® circular connector, IP66/68(3 m;2 h)/69, female, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, female, 3-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Cable diameter: 13.0 - 18.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S B1 ZR3SV GR03

Part No.: 96.031.4554.3

      - Male with screw connection

RST® circular connector, IP66/68(3 m;2 h)/69, male, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, male, 3-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Cable diameter: 13.0 - 18.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S S1 ZR3SV BG03

Part No.: 96.032.4554.3

  - Device connector

    - M16

      - Female with screw connection

RST® device connector M16, IP66/68(3 m;2 h)/69, female, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, female, 3-pole, turned contacts with screw connection fastening screw from inside, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M16x1.5

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S B1 M10 GR03

Part No.: 96.031.6154.3

      - Male with screw connection

RST® device connector M16, IP66/68(3 m;2 h)/69, male, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, male, 3-pole, turned contacts with screw connection fastening screw from inside, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M16x1.5

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S S1 M10 GR03

Part No.: 96.032.6154.3

    - M20

      - Female with screw connection

RST® device connector M20, IP66/68(3 m;2 h)/69, female, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, female, 3-pole, turned contacts with screw connection fastening screw from inside, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M20x1.5

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S B1 M00 GR03

Part No.: 96.031.6054.3

RST® device connector M20, IP66/68(3 m;2 h)/69, female, angled 90°, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, female, angled 90°, 3-pole, turned contacts with screw connection fastening screw from inside, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M20x1.5

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S B1 M00 GR03

Part No.: 96.033.6054.3

      - Male with screw connection

RST® device connector M20, IP66/68(3 m;2 h)/69, male, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, male, 3-pole, turned contacts with screw connection fastening screw from inside, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M20x1.5

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S S1 M00 GR03

Part No.: 96.032.6054.3

RST® device connector M20, IP66/68(3 m;2 h)/69, male, angled 90°, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, male, 3-pole, angled 90°, turned contacts with screw connection fastening screw from inside, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M20x1.5

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S S1 M00 GR03

Part No.: 96.034.6054.3

    - M25

      - Female with screw connection

RST® device connector M25, IP66/68(3 m;2 h)/69, female, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, female, 3-pole, turned contacts with screw connection fastening screw from outside, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M25x1.5

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S B1 M01V GR03

Part No.: 96.031.5054.3

RST® device connector M25, IP66/68(3 m;2 h)/69, female, angled 90°, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, female, angled 90°, 3-pole, turned contacts with screw connection fastening screw from outside, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M25x1.5

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S B1 M01V GR03

Part No.: 96.033.6254.3

      - Male with screw connection

RST® device connector M25, IP66/68(3 m;2 h)/69, male, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, male, 3-pole, turned contacts with screw connection fastening screw from outside, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M25x1.5

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S S1 M01V GR03

Part No.: 96.032.5054.3

RST® device connector M25, IP66/68(3 m;2 h)/69, male, angled 90°, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, male, angled 90°, 3-pole, turned contacts with screw connection fastening screw from outside, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M25x1.5

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I3S S1 M01V GR03

Part No.: 96.034.6254.3

  - Cable assemblies

    - Cable assemblies, 4.0 mm², fire class Eca, containing halogen

RST® extension cable IP66/68(3 m;2 h)/69, female, male, 3x 4.0 mm², 250 V, 25 A, Eca, length: x m, concrete gray/black

System RST® CLASSIC - assembled H05VV-F extension cable with protective ground IP66/68(3 m;2 h)/69, 3-pole, 4.0 mm², ², fire class Eca, with locking, male RST25i3, female RST25i3, according to IEC 61535, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m;2 h)/69

Cable type: H05VV-F

Fire class: Eca

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST25I3KSBS 40P x0BG03

Part No.: 96.834.x000.3 (x corresponds to length in m)

RST® connection cable IP66/68(3 m;2 h)/69, female, free end, 3x 4.0 mm², 250 V, 25 A, Eca, length: x m, concrete gray/black

System RST® CLASSIC - assembled H05VV-F connection cable with protective ground IP66/68(3 m;2 h)/69, 3-pole, 4.0 mm², ², fire class Eca, female RST25i3, free end (ultrasonic welded), according to IEC 61535, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m;2 h)/69

Cable type: H05VV-F

Fire class: Eca

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST25I3KSB- 40P x0BG03

Part No.: 96.834.x003.3 (x corresponds to length in m)

RST® connection cable IP66/68(3 m;2 h)/69, male, free end, 3x 4.0 mm², 250 V, 25 A, Eca, length: x m, concrete gray/black

System RST® CLASSIC - assembled H05VV-F connection cable with protective ground IP66/68(3 m;2 h)/69, 3-pole, 4.0 mm², ², fire class Eca, with locking, male RST25i3, free end (ultrasonic welded), according to IEC 61535, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m;2 h)/69

Cable type: H05VV-F

Fire class: Eca

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST25I3KS-S 40P x0BG03

Part No.: 96.834.x004.3 (x corresponds to length in m)

    - Cable assemblies, 4.0 mm², insulating rubber compound

RST® extension cable IP66/68(3 m;2 h)/69, female, male, 3x 4.0 mm², 250 V, 25 A, length: x m, concrete gray/black

System RST® CLASSIC - assembled H07RN-F extension cable with protective ground IP66/68(3 m;2 h)/69, 3-pole, 4.0 mm², with locking, male RST25i3, female RST25i3, according to IEC 61535, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m;2 h)/69

Cable type: H07RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST25I3KSBS 40 x0BG03

Part No.: 96.834.x030.3 (x corresponds to length in m)

RST® connection cable IP66/68(3 m;2 h)/69, female, free end, 3x 4.0 mm², 250 V, 25 A, length: x m, concrete gray/black

System RST® CLASSIC - assembled H07RN-F connection cable with protective ground IP66/68(3 m;2 h)/69, 3-pole, 4.0 mm², female RST25i3, free end (ultrasonic welded), according to IEC 61535, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m;2 h)/69

Cable type: H07RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST25I3KSB- 40 x0BG03

Part No.: 96.834.x033.3 (x corresponds to length in m)

RST® connection cable IP66/68(3 m;2 h)/69, male, free end, 3x 4.0 mm², 250 V, 25 A, length: x m, concrete gray/black

System RST® CLASSIC - assembled H07RN-F connection cable with protective ground IP66/68(3 m;2 h)/69, 3-pole, 4.0 mm², with locking, male RST25i3, free end (ultrasonic welded), according to IEC 61535, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m;2 h)/69

Cable type: H07RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST25I3KS-S 40 x0BG03

Part No.: 96.834.x034.3 (x corresponds to length in m)

- RST® CLASSIC - Circular connector system IP66/68(3 m;2 h)/69 - 5-pole - RST25i5

  - Connector

    - Cable diameter 6-10 mm

      - Female with screw connection

RST® circular connector, IP66/68(3 m;2 h)/69, female, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, female, 5-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: up to 4.0 mm² (without ferrules)

Cable diameter: 6.0 - 10.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S B1 ZR2S BG03

Part No.: 96.051.4054.3

RST® circular connector, IP66/68(3 m;2 h)/69, splitter connector, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, female with splitter connector (two cable glands), 3-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Terminal cross-section: 0.75 - 2.5 mm² (without ferrules)

Cable diameter: 6.0 - 10.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 2

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5F B2 ZRX GR03

Part No.: 99.584.0000.7

      - Male with screw connection

RST® circular connector, IP66/68(3 m;2 h)/69, male, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, male, 5-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: up to 4.0 mm² (without ferrules)

Cable diameter: 6.0 - 10.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S S1 ZR2SV BG03

Part No.: 96.052.4054.3

    - Cable diameter 10-14 mm

      - Female with screw connection

RST® circular connector, IP66/68(3 m;2 h)/69, female, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, female, 5-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: up to 4.0 mm² (without ferrules)

Cable diameter: 10.0 - 14.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S B1 ZR2S BG03

Part No.: 96.051.4154.3

RST® circular connector, IP66/68(3 m;2 h)/69, splitter connector, 3-pole, 250 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, female with splitter connector (two cable glands), 3-pole, turned contacts with spring clamp connection, compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Terminal cross-section: 0.75 - 2.5 mm² (without ferrules)

Cable diameter: 10.0 - 14.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 2

Marking: L, N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5F B2 ZRX GR03

Part No.: 99.585.0000.7

      - Male with screw connection

RST® circular connector, IP66/68(3 m;2 h)/69, male, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, male, 5-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: up to 4.0 mm² (without ferrules)

Cable diameter: 10.0 - 14.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S S1 ZR2SV BG03

Part No.: 96.052.4154.3

    - Cable diameter 13-18 mm

      - Female with screw connection

RST® circular connector, IP66/68(3 m;2 h)/69, female, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, female, 5-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: up to 4.0 mm² (without ferrules)

Cable diameter: 13.0 - 18.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S B1 ZR3S BG03

Part No.: 96.051.4554.3

RST® circular connector, IP66/68(3 m;2 h)/69, female, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, female, 5-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Cable diameter: 13.0 - 18.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S6B1 ZR3SV GR03

Part No.: 99.575.0000.7

      - Male with screw connection

RST® circular connector, IP66/68(3 m;2 h)/69, male, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, male, 5-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: up to 4.0 mm² (without ferrules)

Cable diameter: 13.0 - 18.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S S1 ZR3SV BG03

Part No.: 96.052.4554.3

RST® circular connector, IP66/68(3 m;2 h)/69, male, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Circular connector according to IEC 61535 for permanent installation with type of protection IP66/68(3 m;2 h)/69, male, 5-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Cable diameter: 13.0 - 18.0 mm (H05VV, H07RN-F)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S6S1 ZR3SV GR03

Part No.: 99.576.0000.7

  - Device connector

    - M16

      - Female with screw connection

RST® device connector M16, IP66/68(3 m;2 h)/69, female, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, female, 5-pole, turned contacts with screw connection fastening screw from inside, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M16x1.5

Terminal cross-section: up to 4.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S B1 M10 GR03

Part No.: 96.051.6154.3

      - Male with screw connection

RST® device connector M16, IP66/68(3 m;2 h)/69, male, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, male, 5-pole, turned contacts with screw connection fastening screw from inside, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M16x1.5

Terminal cross-section: up to 4.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S S1 M10 BG03

Part No.: 96.052.6154.3

    - M20

      - Female with screw connection

RST® device connector M20, IP66/68(3 m;2 h)/69, female, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, female, 5-pole, turned contacts with screw connection fastening screw from inside, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M20x1.5

Terminal cross-section: up to 4.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S B1 M00 GR03

Part No.: 96.051.6054.3

RST® device connector M20, IP66/68(3 m;2 h)/69, female, angled 90°, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, female, angled 90°, 5-pole, turned contacts with screw connection fastening screw from inside, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M20x1.5

Terminal cross-section: up to 4.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S B1 M00 GR03

Part No.: 96.053.6054.3

      - Male with screw connection

RST® device connector M20, IP66/68(3 m;2 h)/69, male, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, male, 5-pole, turned contacts with screw connection fastening screw from inside, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M20x1.5

Terminal cross-section: up to 4.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S S1 M00 BG03

Part No.: 96.052.6054.3

RST® device connector M20, IP66/68(3 m;2 h)/69, male, angled 90°, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, male, angled 90°, 5-pole, turned contacts with screw connection fastening screw from inside, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M20x1.5

Terminal cross-section: up to 4.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S S1 M00 BG03

Part No.: 96.054.6054.3

    - M25

      - Female with screw connection

RST® device connector M25, IP66/68(3 m;2 h)/69, female, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, female, 5-pole, turned contacts with screw connection fastening screw from outside, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M25x1.5

Terminal cross-section: up to 4.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S B1 M01V GR03

Part No.: 96.051.5054.3

RST® device connector M25, IP66/68(3 m;2 h)/69, female, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, female, 5-pole, turned contacts with screw connection fastening screw from outside, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M25x1.5

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25i5S6B1 M01V GR03

Part No.: 99.577.0000.7

RST® device connector M25, IP66/68(3 m;2 h)/69, female, angled 90°, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, female, angled 90°, 5-pole, turned contacts with screw connection fastening screw from outside, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M25x1.5

Terminal cross-section: up to 4.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S B1 M01V GR03

Part No.: 96.053.6254.3

      - Male with screw connection

RST® device connector M25, IP66/68(3 m;2 h)/69, male, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, male, 5-pole, turned contacts with screw connection fastening screw from outside, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M25x1.5

Terminal cross-section: up to 4.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S S1 M01V BG03

Part No.: 96.052.5054.3

RST® device connector M25, IP66/68(3 m;2 h)/69, male, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, male, 5-pole, turned contacts with screw connection fastening screw from outside, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M25x1.5

Terminal cross-section: 0.75 - 6.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25i5S6S1 M01V GR03

Part No.: 99.578.0000.7

RST® device connector M25, IP66/68(3 m;2 h)/69, male, angled 90°, 5-pole, 250/400 V, 25 A, concrete gray

System RST® CLASSIC - Device connector according to IEC 61535 for permanent installation, male, angled 90°, 5-pole, turned contacts with screw connection fastening screw from outside, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3 m;2 h)/69

Thread: M25x1.5

Terminal cross-section: up to 4.0 mm² (without ferrules)

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: L, N, PE, 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST25I5S S1 M01V BG03

Part No.: 96.054.6254.3

  - Cable assemblies

    - Cable assemblies, 4.0 mm², fire class Eca, containing halogen

RST® extension cable IP66/68(3 m;2 h)/69, female, male, 3x 4.0 mm², 250/400 V, 25 A, Eca, length: x m, concrete gray/black

System RST® CLASSIC - assembled H05VV-F extension cable with protective ground IP66/68(3 m;2 h)/69, 5-pole, 4.0 mm², fire class Eca, with locking, male RST25i5, female RST25i5, according to IEC 61535, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m;2 h)/69

Cable type: H05VV-F

Fire class: Eca

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST25I5KSBS 40P x0BG03

Part No.: 96.854.x000.3 (x corresponds to length in m)

RST® connection cable IP66/68(3 m;2 h)/69, female, free end, 3x 4.0 mm², 250/400 V, 25 A, Eca, length: x m, concrete gray/black

System RST® CLASSIC - assembled H05VV-F connection cable with protective ground IP66/68(3 m;2 h)/69, 5-pole, 4.0 mm², fire class Eca, female RST25i5, free end (ultrasonic welded), according to IEC 61535, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m;2 h)/69

Cable type: H05VV-F

Fire class: Eca

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST25I5KSB- 40P x0BG03

Part No.: 96.854.x003.3 (x corresponds to length in m)

RST® connection cable IP66/68(3 m;2 h)/69, male, free end, 3x 4.0 mm², 250/400 V, 25 A, Eca, length: x m, concrete gray/black

System RST® CLASSIC - assembled H05VV-F connection cable with protective ground IP66/68(3 m;2 h)/69, 5-pole, 4.0 mm², fire class Eca, with locking, male RST25i5, free end (ultrasonic welded), according to IEC 61535, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m;2 h)/69

Cable type: H05VV-F

Fire class: Eca

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST25I5KS-S 40P x0BG03

Part No.: 96.854.x004.3 (x corresponds to length in m)

    - Cable assemblies, 4.0 mm², insulating rubber compound

RST® extension cable IP66/68(3 m;2 h)/69, female, male, 3x 4.0 mm², 250/400 V, 25 A, length: x m, concrete gray/black

System RST® CLASSIC - assembled H07RN-F extension cable with protective ground IP66/68(3 m;2 h)/69, 5-pole, 4.0 mm², with locking, male RST25i5, female RST25i5, according to IEC 61535, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m;2 h)/69

Cable type: H07RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST25I5KSBS 40 x0BG03

Part No.: 96.854.x030.3 (x corresponds to length in m)

RST® connection cable IP66/68(3 m;2 h)/69, female, free end, 3x 4.0 mm², 250/400 V, 25 A, length: x m, concrete gray/black

System RST® CLASSIC - assembled H07RN-F connection cable with protective ground IP66/68(3 m;2 h)/69, 5-pole, 4.0 mm², female RST25i5, free end (ultrasonic welded), according to IEC 61535, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m;2 h)/69

Cable type: H07RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST25I5KSB- 40 x0BG03

Part No.: 96.854.x033.3 (x corresponds to length in m)

RST® connection cable IP66/68(3 m;2 h)/69, male, free end, 3x 4.0 mm², 250/400 V, 25 A, length: x m, concrete gray/black

System RST® CLASSIC - assembled H07RN-F connection cable with protective ground IP66/68(3 m;2 h)/69, 5-pole, 4.0 mm², with locking, male RST25i5, free end (ultrasonic welded), according to IEC 61535, mechanical/color coding concrete gray/black.

Rated voltage: 250/400 V

Rated current: 25 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m;2 h)/69

Cable type: H07RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST25I5KS-S 40 x0BG03

Part No.: 96.854.x034.3 (x corresponds to length in m)

96.031.4054.3	2

96.031.4154.3	5

96.031.4253.3	3

96.031.4554.3	7

96.031.5054.3	13

96.031.6054.3	11

96.031.6154.3	9

96.032.4054.3	4

96.032.4154.3	6

96.032.4554.3	8

96.032.5054.3	14

96.032.6054.3	12

96.032.6154.3	10

96.033.4054.3	2

96.033.4154.3	5

96.033.6054.3	11

96.033.6254.3	13

96.034.4054.3	4

96.034.4154.3	6

96.034.6054.3	12

96.034.6254.3	14

96.051.4054.3	19

96.051.4154.3	21

96.051.4554.3	23

96.051.5054.3	29

96.051.6054.3	27

96.051.6154.3	25

96.052.4054.3	20

96.052.4154.3	22

96.052.4554.3	24

96.052.5054.3	31

96.052.6054.3	28

96.052.6154.3	26

96.053.6054.3	27

96.053.6254.3	30

96.054.6054.3	28

96.054.6254.3	32

96.834.x000.3	15

96.834.x003.3	15

96.834.x004.3	16

96.834.x030.3	17

96.834.x033.3	17

96.834.x034.3	18

96.854.x000.3	33

96.854.x003.3	33

96.854.x004.3	34

96.854.x030.3	35

96.854.x033.3	35

96.854.x034.3	36

99.575.0000.7	23

99.576.0000.7	24

99.577.0000.7	29

99.578.0000.7	31

99.584.0000.7	19

99.585.0000.7	21