1	RST® MICRO - Circular connector system IP66/68(3m;2h)/69 - 2-pole - RST08i2	2

1.1	Connector	2

1.1.1	Female with screw connection	2

1.1.2	Male with crew connection	6

1.2	Device connector M14	10

1.2.1	Female with screw connection	10

1.2.2	Male with crew connection	14

1.3	Distribution block	17

1.4	Cable assemblies	20

1.4.1	Cable assemblies, 0.75 mm², containing halogen	20

1.5	Accessories	24

2	RST® MICRO - Circular connector system IP66/68(3m;2h)/69 - 3-pole - RST08i3	25

2.1	Connector	25

2.1.1	Female with screw connection	25

2.1.2	Male with crew connection	29

2.2	Device connector M14	33

2.2.1	Female with screw connection	33

2.2.2	Male with crew connection	37

2.3	Distribution block	40

2.4	Cable assemblies	43

2.4.1	Cable assemblies, 0.75 mm², containing halogen	43

2.5	Accessories	46

3	RST® MICRO - Circular connector system IP66/68(3m;2h)/69 - 4-pole - RST08i4	47

3.1	Connector	47

3.1.1	Female with screw connection	47

3.1.2	Male with crew connection	51

3.2	Device connector M16	55

3.2.1	Female with screw connection	55

3.2.2	Male with crew connection	59

3.3	Accessories	62

4	RST® MICRO - Circular connector system IP66/68(3m;2h)/69 - 5-pole - RST08i5	63

4.1	Connector	63

4.1.1	Female with screw connection	63

4.1.2	Male with crew connection	67

4.2	Device connector M16	71

4.2.1	Female with screw connection	71

4.2.2	Male with crew connection	75

4.3	Accessories	78

- RST® MICRO - Circular connector system IP66/68(3m;2h)/69 - 2-pole - RST08i2

  - Connector

    - Female with screw connection

RST® MICRO circular connector, IP66/68(3m;2h)/69, female, 2-pole, 250/400 V, 8 A, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 2-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 ZR1 SW

Part No.: 41.021.3043.1

RST® MICRO circular connector, IP66/68(3m;2h)/69, female, 2-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 2-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 ZR1 SW

Part No.: 41.02L.3043.1

RST® MICRO circular connector, IP66/68(3m;2h)/69, female, 2-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 2-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 ZR1 BL

Part No.: 41.021.3043.9

RST® MICRO circular connector, IP66/68(3m;2h)/69, female, 2-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 2-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 ZR1 BL

Part No.: 4L.021.3043.9

RST® MICRO circular connector, IP66/68(3m;2h)/69, female, 2-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 2-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 ZR1 SG

Part No.: 41.021.3041.4

RST® MICRO circular connector, IP66/68(3m;2h)/69, female, 2-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 2-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 ZR1 SG

Part No.: 4L.021.3041.4

RST® MICRO circular connector, IP66/68(3m;2h)/69, female, 2-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 2-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 ZR1 GR

Part No.: 41.021.3041.8

RST® MICRO circular connector, IP66/68(3m;2h)/69, female, 2-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 2-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 ZR1 GR

Part No.: 4L.021.3041.8

    - Male with crew connection

RST® MICRO circular connector, IP66/68(3m;2h)/69, male, 2-pole, 250/400 V, 8 A, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 2-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 ZR1 SW

Part No.: 41.022.3043.1

RST® MICRO circular connector, IP66/68(3m;2h)/69, male, 2-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 2-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 ZR1 SW

Part No.: 4L.022.3043.1

RST® MICRO circular connector, IP66/68(3m;2h)/69, male, 2-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 2-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 ZR1 BL

Part No.: 41.022.3043.9

RST® MICRO circular connector, IP66/68(3m;2h)/69, male, 2-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 2-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 ZR1 BL

Part No.: 4L.022.3043.9

RST® MICRO circular connector, IP66/68(3m;2h)/69, male, 2-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 2-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 ZR1 SG

Part No.: 41.022.3041.4

RST® MICRO circular connector, IP66/68(3m;2h)/69, male, 2-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 2-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 ZR1 SG

Part No.: 4L.022.3041.4

RST® MICRO circular connector, IP66/68(3m;2h)/69, male, 2-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 2-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 ZR1 GR

Part No.: 41.022.3041.8

RST® MICRO circular connector, IP66/68(3m;2h)/69, male, 2-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 2-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 ZR1 GR

Part No.: 4L.022.3041.8

  - Device connector M14

    - Female with screw connection

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 2-pole, 250/400 V, 8 A, black

System RST® MICRO - Device connector, female, 2-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 M00 SW

Part No.: 41.021.4043.1

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 2-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Device connector, female, 2-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 M00 SW

Part No.: 4L.021.4043.1

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 2-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Device connector, female, 2-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 M00 BL

Part No.: 41.021.4043.9

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 2-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Device connector, female, 2-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 M00 BL

Part No.: 4L.021.4043.9

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 2-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Device connector, female, 2-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 M00 SG

Part No.: 41.021.4041.4

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 2-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Device connector, female, 2-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 M00 SG

Part No.: 4L.021.4041.4

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 2-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Device connector, female, 2-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 M00 GR

Part No.: 41.021.4041.8

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 2-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Device connector, female, 2-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S B1 M00 GR

Part No.: 4L.021.4041.8

    - Male with crew connection

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 2-pole, 250/400 V, 8 A, black

System RST® MICRO - Device connector, male, 2-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 M00 SW

Part No.: 41.022.4043.1

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 2-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Device connector, male, 2-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 M00 SW

Part No.: 4L.022.4043.1

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 2-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Device connector, male, 2-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 M00 BL

Part No.: 41.022.4043.9

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 2-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Device connector, male, 2-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 M00 BL

Part No.: 4L.022.4043.9

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 2-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Device connector, male, 2-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 M00 SG

Part No.: 41.022.4041.4

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 2-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Device connector, male, 2-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 M00 SG

Part No.: 4L.022.4041.4

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 2-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Device connector, male, 2-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 M00 GR

Part No.: 41.022.4041.8

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 2-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Device connector, male, 2-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I2S S1 M00 GR

Part No.: 4L.022.4041.8

  - Distribution block

RST® distribution block, IP66/68(3 m; 2 h)/69, 2-pole, 1 input, 2 outputs, with mounting option, 250/400 V, 8 A, black

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 2 outputs, 2-pole, with mounting option on distributor mounting plate, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 2

Halogen-free: yes

Dimension: 37x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I2B 2P1 F VG SW

Part No.: 4L.020.1143.1

RST® distribution block, IP66/68(3 m; 2 h)/69, 2-pole, 1 input, 2 outputs, with mounting option, 250/400 V, 8 A, light blue

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 2 outputs, 2-pole, with mounting option on distributor mounting plate, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 2

Halogen-free: yes

Dimension: 37x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I2B 2P1 F VG BL

Part No.: 4L.020.1143.9

RST® distribution block, IP66/68(3 m; 2 h)/69, 2-pole, 1 input, 2 outputs, with mounting option, ~50/120 V, 8 A, slate grey

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 2 outputs, 2-pole, with mounting option on distributor mounting plate, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 2

Halogen-free: yes

Dimension: 37x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I2B 2P1 F VG SG

Part No.: 4L.020.1141.4

RST® distribution block, IP66/68(3 m; 2 h)/69, 2-pole, 1 input, 2 outputs, series connection, with mounting option, ~50/120 V, 8 A, pebble grey

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 2 outputs, 2-pole, series connection, with mounting option on distributor mounting plate, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 2, series connection

Halogen-free: yes

Dimension: 37x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I2B 2P1 F VG SG

Part No.: 4L.020.1241.8

RST® distribution block, IP66/68(3 m; 2 h)/69, 2-pole, 1 input, 3 outputs, with mounting option, 250/400 V, 8 A, black

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 3 outputs, 2-pole, with mounting option on distributor mounting plate, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 3

Halogen-free: yes

Dimension: 47x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I2B 3P1 F VG SW

Part No.: 4L.020.0143.1

RST® distribution block, IP66/68(3 m; 2 h)/69, 2-pole, 1 input, 3 outputs, with mounting option, 250/400 V, 8 A, light blue

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 3 outputs, 2-pole, with mounting option on distributor mounting plate, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 3

Halogen-free: yes

Dimension: 47x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I2B 3P1 F VG BL

Part No.: 4L.020.0143.9

RST® distribution block, IP66/68(3 m; 2 h)/69, 2-pole, 1 input, 3 outputs, with mounting option, ~50/120 V, 8 A, slate grey

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 3 outputs, 2-pole, with mounting option on distributor mounting plate, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 3

Halogen-free: yes

Dimension: 47x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I2B 3P1 F VG SG

Part No.: 4L.020.0141.4

RST® distribution block, IP66/68(3 m; 2 h)/69, 2-pole, 1 input, 3 outputs, series connection, with mounting option, ~50/120 V, 8 A, pebble grey

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 3 outputs, 2-pole, series connection, with mounting option on distributor mounting plate, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 3, series connection

Halogen-free: yes

Dimension: 47x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I2B 3P1 F VG SG

Part No.: 4L.020.0241.8

  - Cable assemblies

    - Cable assemblies, 0.75 mm², containing halogen

RST® MICRO extension cable IP66/68(3m;2h)/69, female, male, 2x 0.75 mm², 250/400 V, 6 A, length: x m, black

System RST® MICRO - assembled H05RN-F extension cable IP66/68(3m;2h)/69, 2-pole, 0.75 mm², with locking, male RST08i2, female RST08i2, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I2 BS 08P x0SW

Part No.: 41.427.x030.1 (x corresponds to length in m)

RST® MICRO extension cable IP66/68(3m;2h)/69, female, male, 2x 0.75 mm², 250/400 V, 6 A, length: x m, light blue

System RST® MICRO - assembled H05RN-F extension cable IP66/68(3m;2h)/69, 2-pole, 0.75 mm², with locking, male RST08i2, female RST08i2, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I2 BS 08P x0BL

Part No.: 41.427.x030.9 (x corresponds to length in m)

RST® MICRO extension cable IP66/68(3m;2h)/69, female, male, 2x 0.75 mm², ~50/-120 V, 6 A, length: x m, slate grey

System RST® MICRO - assembled H05RN-F extension cable IP66/68(3m;2h)/69, 2-pole, 0.75 mm², with locking, male RST08i2, female RST08i2, mechanical/color coding slate grey.

Rated voltage: ~50/-120 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I2 BS 08P x0SG

Part No.: 41.427.x032.4 (x corresponds to length in m)

RST® MICRO extension cable IP66/68(3m;2h)/69, female, male, 2x 0.75 mm², ~50/-120 V, 6 A, length: x m, pebble grey

System RST® MICRO - assembled H05RN-F extension cable IP66/68(3m;2h)/69, 2-pole, 0.75 mm², with locking, male RST08i2, female RST08i2, mechanical/color coding pebble grey.

Rated voltage: ~50/-120 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I2 BS 08P x0GR

Part No.: 41.427.x032.8 (x corresponds to length in m)

RST® MICRO connection cable IP66/68(3m;2h)/69, female, free end, 2x 0.75 mm², 250/400 V, 6 A, length: x m, black

System RST® MICRO - assembled H05RN-F connection cable IP66/68(3m;2h)/69, 2-pole, 0.75 mm², with locking, female RST08i2, free end (ultrasonic welded), mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I2 B- 08P x0SW

Part No.: 41.427.x033.1 (x corresponds to length in m)

RST® MICRO connection cable IP66/68(3m;2h)/69, female, free end, 2x 0.75 mm², 250/400 V, 6 A, length: x m, light blue

System RST® MICRO - assembled H05RN-F connection cable IP66/68(3m;2h)/69, 2-pole, 0.75 mm², with locking, female RST08i2, free end (ultrasonic welded), mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I2 B- 08P x0BL

Part No.: 41.427.x033.9 (x corresponds to length in m)

RST® MICRO connection cable IP66/68(3m;2h)/69, female, free end, 2x 0.75 mm², ~50/-120 V, 6 A, length: x m, slate grey

System RST® MICRO - assembled H05RN-F connection cable IP66/68(3m;2h)/69, 2-pole, 0.75 mm², with locking, female RST08i2, free end (ultrasonic welded), mechanical/color coding slate grey.

Rated voltage: ~50/-120 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I2 B- 08P x0SG

Part No.: 41.427.x037.4 (x corresponds to length in m)

RST® MICRO connection cable IP66/68(3m;2h)/69, female, free end, 2x 0.75 mm², ~50/-120 V, 6 A, length: x m, pebble grey

System RST® MICRO - assembled H05RN-F connection cable IP66/68(3m;2h)/69, 2-pole, 0.75 mm², with locking, female RST08i2, free end (ultrasonic welded), mechanical/color coding pebble grey.

Rated voltage: ~50/-120 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I2 B- 08P x0GR

Part No.: 41.427.x037.8 (x corresponds to length in m)

RST® MICRO connection cable IP66/68(3m;2h)/69, male, free end, 2x 0.75 mm², 250/400 V, 6 A, length: x m, black

System RST® MICRO - assembled H05RN-F connection cable IP66/68(3m;2h)/69, 2-pole, 0.75 mm², with locking, male RST08i2, free end (ultrasonic welded), mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I2 -S 08P x0SW

Part No.: 41.427.x034.1 (x corresponds to length in m)

RST® MICRO connection cable IP66/68(3m;2h)/69, male, free end, 2x 0.75 mm², 250/400 V, 6 A, length: x m, light blue

System RST® MICRO - assembled H05RN-F connection cable IP66/68(3m;2h)/69, 2-pole, 0.75 mm², with locking, male RST08i2, free end (ultrasonic welded), mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I2 -S 08P x0BL

Part No.: 41.427.x034.9 (x corresponds to length in m)

RST® MICRO connection cable IP66/68(3m;2h)/69, male, free end, 2x 0.75 mm², ~50/-120 V, 6 A, length: x m, slate grey

System RST® MICRO - assembled H05RN-F connection cable IP66/68(3m;2h)/69, 2-pole, 0.75 mm², with locking, male RST08i2, free end (ultrasonic welded), mechanical/color coding slate grey.

Rated voltage: ~50/-120 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I2 -S 08P x0SG

Part No.: 41.427.x038.4 (x corresponds to length in m)

RST® MICRO connection cable IP66/68(3m;2h)/69, male, free end, 2x 0.75 mm², ~50/-120 V, 6 A, length: x m, pebble grey

System RST® MICRO - assembled H05RN-F connection cable IP66/68(3m;2h)/69, 2-pole, 0.75 mm², with locking, male RST08i2, free end (ultrasonic welded), mechanical/color coding pebble grey.

Rated voltage: ~50/-120 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I2 -S 08P x0GR

Part No.: 41.427.x038.8 (x corresponds to length in m)

  - Accessories

RST® MICRO cover piece IP66/68(3m;2h)/69 for female, 2-/3-pole, captive against loss, black

System RST® MICRO - Cover piece IP66/68(3m;2h)/69 for female parts RST08, 2-/3-pole, with mounting strap for snapping on, black.

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08 S VS SW

Part No.: 06.561.5053.1

RST® MICRO cover piece IP66/68(3m;2h)/69 for male, 2-/3-pole, captive against loss, black

System RST® MICRO - Cover piece IP66/68(3m;2h)/69 for male parts RST08, 2-/3-pole, with mounting strap for snapping on, black.

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08 B VS SW

Part No.: 06.563.5153.1

RST® MICRO Pin wrench for screw terminals

System RST® MICRO - Pin wrench for RST® MICRO screw terminals.

Brand: Wieland Electric GmbH

Type: RST08 S

Part No.: 06.502.6100.0

RST® MICRO Hexagonal wrench for screw terminals

System RST® MICRO - Hexagonal wrench for RST® MICRO screw terminals.

Brand: Wieland Electric GmbH

Type: RST08 S

Part No.: 06.502.6300.0

- RST® MICRO - Circular connector system IP66/68(3m;2h)/69 - 3-pole - RST08i3

  - Connector

    - Female with screw connection

RST® Circular connector, IP66/68(3m;2h)/69, female, 3-pole, 250/400 V, 8 A, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 3-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 ZR1 SW

Part No.: 41.031.3053.1

RST® Circular connector, IP66/68(3m;2h)/69, female, 3-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 3-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 ZR1 SW

Part No.: 4L.031.3053.1

RST® Circular connector, IP66/68(3m;2h)/69, female, 3-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 3-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 ZR1 BL

Part No.: 41.031.3043.9

RST® Circular connector, IP66/68(3m;2h)/69, female, 3-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 3-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 ZR1 BL

Part No.: 4L.031.3043.9

RST® Circular connector, IP66/68(3m;2h)/69, female, 3-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 3-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 ZR1 SG

Part No.: 41.031.3041.4

RST® Circular connector, IP66/68(3m;2h)/69, female, 3-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 3-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 ZR1 SG

Part No.: 4L.031.3041.4

RST® Circular connector, IP66/68(3m;2h)/69, female, 3-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 3-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 ZR1 GR

Part No.: 41.031.3041.8

RST® Circular connector, IP66/68(3m;2h)/69, female, 3-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 3-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 ZR1 GR

Part No.: 4L.031.3041.8

    - Male with crew connection

RST® Circular connector, IP66/68(3m;2h)/69, male, 3-pole, 250/400 V, 8 A, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 3-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 ZR1 SW

Part No.: 41.032.3053.1

RST® Circular connector, IP66/68(3m;2h)/69, male, 3-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 3-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 ZR1 SW

Part No.: 4L.032.3053.1

RST® Circular connector, IP66/68(3m;2h)/69, male, 3-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 3-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 ZR1 BL

Part No.: 41.032.3043.9

RST® Circular connector, IP66/68(3m;2h)/69, male, 3-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 3-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 ZR1 BL

Part No.: 4L.032.3043.9

RST® Circular connector, IP66/68(3m;2h)/69, male, 3-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 3-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 ZR1 SG

Part No.: 41.032.3041.4

RST® Circular connector, IP66/68(3m;2h)/69, male, 3-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 3-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 ZR1 SG

Part No.: 4L.032.3041.4

RST® Circular connector, IP66/68(3m;2h)/69, male, 3-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 3-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 ZR1 GR

Part No.: 41.032.3041.8

RST® Circular connector, IP66/68(3m;2h)/69, male, 3-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 3-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 4.0 - 7.0 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 ZR1 GR

Part No.: 4L.032.3041.8

  - Device connector M14

    - Female with screw connection

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 3-pole, 250/400 V, 8 A, black

System RST® MICRO - Device connector, female, 3-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 M00 SW

Part No.: 41.031.4053.1

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 3-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Device connector, female, 3-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 M00 SW

Part No.: 4L.031.4053.1

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 3-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Device connector, female, 3-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 M00 BL

Part No.: 41.031.4043.9

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 3-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Device connector, female, 3-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 M00 BL

Part No.: 4L.031.4043.9

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 3-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Device connector, female, 3-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 M00 SG

Part No.: 41.031.4041.4

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 3-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Device connector, female, 3-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 M00 SG

Part No.: 4L.031.4041.4

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 3-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Device connector, female, 3-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 M00 GR

Part No.: 41.031.4041.8

RST® MICRO device connector M14, IP66/68(3m;2h)/69, female, 3-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Device connector, female, 3-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S B1 M00 GR

Part No.: 4L.031.4041.8

    - Male with crew connection

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 3-pole, 250/400 V, 8 A, black

System RST® MICRO - Device connector, male, 3-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 M00 SW

Part No.: 41.032.4053.1

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 3-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Device connector, male, 3-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 M00 SW

Part No.: 4L.032.4053.1

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 3-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Device connector, male, 3-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 M00 BL

Part No.: 41.032.4043.9

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 3-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Device connector, male, 3-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 M00 BL

Part No.: 4L.032.4043.9

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 3-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Device connector, male, 3-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 M00 SG

Part No.: 41.032.4041.4

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 3-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Device connector, male, 3-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 M00 SG

Part No.: 4L.032.4041.4

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 3-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Device connector, male, 3-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 M00 GR

Part No.: 41.032.4041.8

RST® MICRO device connector M14, IP66/68(3m;2h)/69, male, 3-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Device connector, male, 3-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I3S S1 M00 GR

Part No.: 4L.032.4041.8

  - Distribution block

RST® distribution block, IP66/68(3 m; 2 h)/69, 2-pole, 1 input, 2 outputs, with mounting option, 250/400 V, 8 A, black

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 2 outputs, 3-pole, with mounting option on distributor mounting plate, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 2

Halogen-free: yes

Dimension: 37x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I3B 2P1 F VG SW

Part No.: 4L.030.1143.1

RST® distribution block, IP66/68(3 m; 2 h)/69, 3-pole, 1 input, 2 outputs, with mounting option, 250/400 V, 8 A, light blue

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 2 outputs, 3-pole, with mounting option on distributor mounting plate, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 2

Halogen-free: yes

Dimension: 37x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I3B 2P1 F VG BL

Part No.: 4L.030.1143.9

RST® distribution block, IP66/68(3 m; 2 h)/69, 3-pole, 1 input, 2 outputs, with mounting option, ~50/120 V, 8 A, slate grey

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 2 outputs, 3-pole, with mounting option on distributor mounting plate, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 2

Halogen-free: yes

Dimension: 37x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I3B 2P1 F VG SG

Part No.: 4L.030.1141.4

RST® distribution block, IP66/68(3 m; 2 h)/69, 3-pole, 1 input, 2 outputs, series connection, with mounting option, ~50/120 V, 8 A, pebble grey

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 2 outputs, 3-pole, series connection, with mounting option on distributor mounting plate, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 2, series connection

Halogen-free: yes

Dimension: 37x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I3B 2P1 F VG SG

Part No.: 4L.030.1341.8

RST® distribution block, IP66/68(3 m; 2 h)/69, 3-pole, 1 input, 3 outputs, with mounting option, 250/400 V, 8 A, black

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 3 outputs, 3-pole, with mounting option on distributor mounting plate, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 3

Halogen-free: yes

Dimension: 47x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I3B 3P1 F VG SW

Part No.: 4L.030.0143.1

RST® distribution block, IP66/68(3 m; 2 h)/69, 3-pole, 1 input, 3 outputs, with mounting option, 250/400 V, 8 A, light blue

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 3 outputs, 3-pole, with mounting option on distributor mounting plate, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 3

Halogen-free: yes

Dimension: 47x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I3B 3P1 F VG BL

Part No.: 4L.030.0143.9

RST® distribution block, IP66/68(3 m; 2 h)/69, 3-pole, 1 input, 3 outputs, with mounting option, ~50/120 V, 8 A, slate grey

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 3 outputs, 3-pole, with mounting option on distributor mounting plate, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 3

Halogen-free: yes

Dimension: 47x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I3B 3P1 F VG SG

Part No.: 4L.030.0141.4

RST® distribution block, IP66/68(3 m; 2 h)/69, 3-pole, 1 input, 3 outputs, series connection, with mounting option, ~50/120 V, 8 A, pebble grey

System RST® MICRO - distribution block for circular connector IP66/68(3 m; 2 h)/69, 1 input, 3 outputs, 3-pole, series connection, with mounting option on distributor mounting plate, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: IP66/68(3 m; 2 h)/69

Housing material: PA66, V2

Sealing material: NBR

Input: 1

Outputs: 3, series connection

Halogen-free: yes

Dimension: 47x50.6x22.8 mm

Brand: Wieland Electric GmbH

Type: RST08I3B 3P1 F VG SG

Part No.: 4L.030.0341.8

  - Cable assemblies

    - Cable assemblies, 0.75 mm², containing halogen

RST® MICRO extension cable IP66/68(3m;2h)/69, female, male, 3x 0.75 mm², 250/400 V, 6 A, length: x m, black

System RST® MICRO - assembled H05RN-F extension cable IP66/68(3m;2h)/69, 3-pole, 0.75 mm², with locking, male RST08i3, female RST08i3, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I3 BS 08P x0SW

Part No.: 41.437.x030.1 (x corresponds to length in m)

RST® MICRO extension cable IP66/68(3m;2h)/69, female, male, 3x 0.75 mm², 250/400 V, 6 A, length: x m, with contact seal, black

System RST® MICRO - assembled H05RN-F extension cable IP66/68(3m;2h)/69, 3-pole, 0.75 mm², with locking, with contact seal, male RST08i3, female RST08i3, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I3 BS 08P x0SW

Part No.: 4L.437.x030.1 (x corresponds to length in m)

RST® MICRO extension cable IP66/68(3m;2h)/69, female, male, 3x 0.75 mm², ~50/-120 V, 6 A, length: x m, pebble grey

System RST® MICRO - assembled H05RN-F extension cable IP66/68(3m;2h)/69, 3-pole, 0.75 mm², with locking, male RST08i3, female RST08i3, mechanical/color coding pebble grey.

Rated voltage: ~50/-120 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I3 BS 08P x0GR

Part No.: 41.437.x032.8 (x corresponds to length in m)

RST® MICRO connection cable IP66/68(3m;2h)/69, female, free end, 3x 0.75 mm², 250/400 V, 6 A, length: x m, black

System RST® MICRO - assembled H05RN-F connection cable IP66/68(3m;2h)/69, 3-pole, 0.75 mm², with locking, female RST08i3, free end (ultrasonic welded), mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I3 B- 08P x0SW

Part No.: 41.437.x033.1 (x corresponds to length in m)

RST® MICRO connection cable IP66/68(3m;2h)/69, female, free end, 3x 0.75 mm², 250/400 V, 6 A, length: x m, with contact seal, black

System RST® MICRO - assembled H05RN-F connection cable IP66/68(3m;2h)/69, 3-pole, 0.75 mm², with locking, with contact seal, female RST08i3, free end (ultrasonic welded), mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I3 B- 08P x0SW

Part No.: 4L.437.x033.1 (x corresponds to length in m)

RST® MICRO connection cable IP66/68(3m;2h)/69, female, free end, 3x 0.75 mm², ~50/-120 V, 6 A, length: x m, pebble grey

System RST® MICRO - assembled H05RN-F connection cable IP66/68(3m;2h)/69, 3-pole, 0.75 mm², with locking, female RST08i3, free end (ultrasonic welded), mechanical/color coding pebble grey.

Rated voltage: ~50/-120 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I3 B- 08P x0GR

Part No.: 41.437.x037.8 (x corresponds to length in m)

RST® MICRO connection cable IP66/68(3m;2h)/69, male, free end, 3x 0.75 mm², 250/400 V, 6 A, length: x m, black

System RST® MICRO - assembled H05RN-F connection cable IP66/68(3m;2h)/69, 3-pole, 0.75 mm², with locking, male RST08i3, free end (ultrasonic welded), mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I3 -S 08P x0SW

Part No.: 41.437.x034.1 (x corresponds to length in m)

RST® MICRO connection cable IP66/68(3m;2h)/69, male, free end, 3x 0.75 mm², 250/400 V, 6 A, length: x m, with contact seal, black

System RST® MICRO - assembled H05RN-F connection cable IP66/68(3m;2h)/69, 3-pole, 0.75 mm², with locking, with contact seal, male RST08i3, free end (ultrasonic welded), mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I3 -S 08P x0SW

Part No.: 4L.437.x034.1 (x corresponds to length in m)

RST® MICRO connection cable IP66/68(3m;2h)/69, male, free end, 3x 0.75 mm², ~50/-120 V, 6 A, length: x m, pebble grey

System RST® MICRO - assembled H05RN-F connection cable IP66/68(3m;2h)/69, 3-pole, 0.75 mm², with locking, male RST08i3, free end (ultrasonic welded), mechanical/color coding pebble grey.

Rated voltage: ~50/-120 V

Rated current: 6 A

Type of protection: IP66/68(3m;2h)/69

Cable type: H05RN-F

Cable color: black

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: RST08I3 -S 08P x0GR

Part No.: 41.437.x038.8 (x corresponds to length in m)

  - Accessories

RST® MICRO cover piece IP66/68(3m;2h)/69 for female, 2-/3-pole, captive against loss, black

System RST® MICRO - Cover piece IP66/68(3m;2h)/69 for female parts RST08, 2-/3-pole, with mounting strap for snapping on, black.

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08 S VS SW

Part No.: 06.561.5053.1

RST® MICRO cover piece IP66/68(3m;2h)/69 for male, 2-/3-pole, captive against loss, black

System RST® MICRO - Cover piece IP66/68(3m;2h)/69 for male parts RST08, 2-/3-pole, with mounting strap for snapping on, black.

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08 B VS SW

Part No.: 06.563.5153.1

RST® MICRO Pin wrench for screw terminals

System RST® MICRO - Pin wrench for RST® MICRO screw terminals.

Brand: Wieland Electric GmbH

Type: RST08 S

Part No.: 06.502.6100.0

RST® MICRO Hexagonal wrench for screw terminals

System RST® MICRO - Hexagonal wrench for RST® MICRO screw terminals.

Brand: Wieland Electric GmbH

Type: RST08 S

Part No.: 06.502.6300.0

- RST® MICRO - Circular connector system IP66/68(3m;2h)/69 - 4-pole - RST08i4

  - Connector

    - Female with screw connection

RST® Circular connector, IP66/68(3m;2h)/69, female, 4-pole, 250/400 V, 8 A, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 4-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, PE, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 ZR1 SW

Part No.: 41.041.3053.1

RST® Circular connector, IP66/68(3m;2h)/69, female, 4-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 4-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, PE, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 ZR1 SW

Part No.: 4L.041.3053.1

RST® Circular connector, IP66/68(3m;2h)/69, female, 4-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 4-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 ZR1 BL

Part No.: 41.041.3043.9

RST® Circular connector, IP66/68(3m;2h)/69, female, 4-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 4-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 ZR1 BL

Part No.: 4L.041.3043.9

RST® Circular connector, IP66/68(3m;2h)/69, female, 4-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 4-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 ZR1 GR

Part No.: 41.041.3041.8

RST® Circular connector, IP66/68(3m;2h)/69, female, 4-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 4-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 ZR1 GR

Part No.: 4L.041.3041.8

RST® Circular connector, IP66/68(3m;2h)/69, female, 4-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 4-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 ZR1 SG

Part No.: 41.041.3041.4

RST® Circular connector, IP66/68(3m;2h)/69, female, 4-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 4-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 ZR1 SG

Part No.: 4L.041.3041.4

    - Male with crew connection

RST® Circular connector, IP66/68(3m;2h)/69, male, 4-pole, 250/400 V, 8 A, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 4-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, PE, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 ZR1 SW

Part No.: 41.042.3053.1

RST® Circular connector, IP66/68(3m;2h)/69, male, 4-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 4-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, PE, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 ZR1 SW

Part No.: 4L.042.3053.1

RST® Circular connector, IP66/68(3m;2h)/69, male, 4-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 4-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 ZR1 BL

Part No.: 41.042.3043.9

RST® Circular connector, IP66/68(3m;2h)/69, male, 4-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 4-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 ZR1 BL

Part No.: 4L.042.3043.9

RST® Circular connector, IP66/68(3m;2h)/69, male, 4-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 4-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 ZR1 GR

Part No.: 41.042.3041.8

RST® Circular connector, IP66/68(3m;2h)/69, male, 4-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 4-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 ZR1 GR

Part No.: 4L.042.3041.8

RST® Circular connector, IP66/68(3m;2h)/69, male, 4-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 4-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 ZR1 SG

Part No.: 41.042.3041.4

RST® Circular connector, IP66/68(3m;2h)/69, male, 4-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 4-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 ZR1 SG

Part No.: 4L.042.3041.4

  - Device connector M16

    - Female with screw connection

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 4-pole, 250/400 V, 8 A, black

System RST® MICRO - Device connector, female, 4-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, PE, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 M00 SW

Part No.: 41.041.4053.1

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 4-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Device connector, female, 4-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, PE, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 M00 SW

Part No.: 4L.041.4053.1

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 4-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Device connector, female, 4-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 M00 BL

Part No.: 41.041.4043.9

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 4-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Device connector, female, 4-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 M00 BL

Part No.: 4L.041.4043.9

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 4-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Device connector, female, 4-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 M00 GR

Part No.: 41.041.4041.8

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 4-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Device connector, female, 4-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 M00 GR

Part No.: 4L.041.4041.8

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 4-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Device connector, female, 4-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 M00 SG

Part No.: 41.041.4041.4

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 4-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Device connector, female, 4-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S B1 M00 SG

Part No.: 4L.041.4041.4

    - Male with crew connection

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 4-pole, 250/400 V, 8 A, black

System RST® MICRO - Device connector, male, 4-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, PE, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 M00 SW

Part No.: 41.042.4053.1

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 4-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Device connector, male, 4-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, PE, 2/N, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 M00 SW

Part No.: 4L.042.4053.1

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 4-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Device connector, male, 4-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 M00 BL

Part No.: 41.042.4043.9

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 4-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Device connector, male, 4-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 M00 BL

Part No.: 4L.042.4043.9

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 4-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Device connector, male, 4-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 M00 GR

Part No.: 41.042.4041.8

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 4-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Device connector, male, 4-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 M00 GR

Part No.: 4L.042.4041.8

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 4-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Device connector, male, 4-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 M00 SG

Part No.: 41.042.4041.4

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 4-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Device connector, male, 4-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I4S S1 M00 SG

Part No.: 4L.042.4041.4

  - Accessories

RST® MICRO cover piece IP66/68(3m;2h)/69 for female, 4-/5-pole, captive against loss, black

System RST® MICRO - Cover piece IP66/68(3m;2h)/69 for female parts RST08, 4-/5-pole, with mounting strap for snapping on, black.

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08 S VS SW

Part No.: 06.564.1553.1

RST® MICRO cover piece IP66/68(3m;2h)/69 for male, 4-/5-pole, captive against loss, black

System RST® MICRO - Cover piece IP66/68(3m;2h)/69 for male parts RST08, 4-/5-pole, with mounting strap for snapping on, black.

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08 B VS SW

Part No.: 06.564.1653.1

RST® MICRO Pin wrench for screw terminals

System RST® MICRO - Pin wrench for RST® MICRO screw terminals.

Brand: Wieland Electric GmbH

Type: RST08 S

Part No.: 06.502.6100.0

RST® MICRO Hexagonal wrench for screw terminals

System RST® MICRO - Hexagonal wrench for RST® MICRO screw terminals.

Brand: Wieland Electric GmbH

Type: RST08 S

Part No.: 06.502.6300.0

- RST® MICRO - Circular connector system IP66/68(3m;2h)/69 - 5-pole - RST08i5

  - Connector

    - Female with screw connection

RST® Circular connector, IP66/68(3m;2h)/69, female, 5-pole, 250/400 V, 8 A, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 5-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 ZR1 SW

Part No.: 41.051.3053.1

RST® Circular connector, IP66/68(3m;2h)/69, female, 5-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 5-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 ZR1 SW

Part No.: 4L.051.3053.1

RST® Circular connector, IP66/68(3m;2h)/69, female, 5-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 5-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 ZR1 BL

Part No.: 41.051.3043.9

RST® Circular connector, IP66/68(3m;2h)/69, female, 5-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 5-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 ZR1 BL

Part No.: 4L.051.3043.9

RST® Circular connector, IP66/68(3m;2h)/69, female, 5-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 5-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 ZR1 GR

Part No.: 41.051.3041.8

RST® Circular connector, IP66/68(3m;2h)/69, female, 5-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 5-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 ZR1 GR

Part No.: 4L.051.3041.8

RST® Circular connector, IP66/68(3m;2h)/69, female, 5-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 5-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 ZR1 SG

Part No.: 41.051.3041.4

RST® Circular connector, IP66/68(3m;2h)/69, female, 5-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, female, 5-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 ZR1 SG

Part No.: 4L.051.3041.4

    - Male with crew connection

RST® Circular connector, IP66/68(3m;2h)/69, male, 5-pole, 250/400 V, 8 A, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 5-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 ZR1 SW

Part No.: 41.052.3053.1

RST® Circular connector, IP66/68(3m;2h)/69, male, 5-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 5-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 ZR1 SW

Part No.: 4L.052.3053.1

RST® Circular connector, IP66/68(3m;2h)/69, male, 5-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 5-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 ZR1 BL

Part No.: 41.052.3043.9

RST® Circular connector, IP66/68(3m;2h)/69, male, 5-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 5-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 ZR1 BL

Part No.: 4L.052.3043.9

RST® Circular connector, IP66/68(3m;2h)/69, male, 5-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 5-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 ZR1 GR

Part No.: 41.052.3041.8

RST® Circular connector, IP66/68(3m;2h)/69, male, 5-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 5-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 ZR1 GR

Part No.: 4L.052.3041.8

RST® Circular connector, IP66/68(3m;2h)/69, male, 5-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 5-pole, turned contacts with screw connection, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 ZR1 SG

Part No.: 41.052.3041.4

RST® Circular connector, IP66/68(3m;2h)/69, male, 5-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Circular connector with type of protection IP66/68(3m;2h)/69, male, 5-pole, turned contacts with screw connection, with contact seal, very compact housing with strain relief, field assembly, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Cable diameter: 5.0 - 10.6 mm

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 ZR1 SG

Part No.: 4L.052.3041.4

  - Device connector M16

    - Female with screw connection

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 5-pole, 250/400 V, 8 A, black

System RST® MICRO - Device connector, female, 5-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 M00 SW

Part No.: 41.051.4053.1

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 5-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Device connector, female, 5-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 M00 SW

Part No.: 4L.051.4053.1

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 5-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Device connector, female, 5-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 M00 BL

Part No.: 41.051.4043.9

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 5-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Device connector, female, 5-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 M00 BL

Part No.: 4L.051.4043.9

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 5-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Device connector, female, 5-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 M00 GR

Part No.: 41.051.4041.8

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 5-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Device connector, female, 5-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 M00 GR

Part No.: 4L.051.4041.8

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 5-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Device connector, female, 5-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 M00 SG

Part No.: 41.051.4041.4

RST® MICRO device connector M16, IP66/68(3m;2h)/69, female, 5-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Device connector, female, 5-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S B1 M00 SG

Part No.: 4L.051.4041.4

    - Male with crew connection

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 5-pole, 250/400 V, 8 A, black

System RST® MICRO - Device connector, male, 5-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 M00 SW

Part No.: 41.052.4053.1

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 5-pole, 250/400 V, 8 A, with contact seal, black

System RST® MICRO - Device connector, male, 5-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Wall thicknesses: up to 8 mm

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 M00 SW

Part No.: 4L.052.4053.1

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 5-pole, 250/400 V, 8 A, light blue

System RST® MICRO - Device connector, male, 5-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 M00 BL

Part No.: 41.052.4043.9

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 5-pole, 250/400 V, 8 A, with contact seal, light blue

System RST® MICRO - Device connector, male, 5-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding light blue.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2/N, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 M00 BL

Part No.: 4L.052.4043.9

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 5-pole, ~50/120 V, 8 A, pebble grey

System RST® MICRO - Device connector, male, 5-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 M00 GR

Part No.: 41.052.4041.8

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 5-pole, ~50/120 V, 8 A, with contact seal, pebble grey

System RST® MICRO - Device connector, male, 5-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding pebble grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, PE

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 M00 GR

Part No.: 4L.052.4041.8

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 5-pole, ~50/120 V, 8 A, slate grey

System RST® MICRO - Device connector, male, 5-pole, turned contacts with screw connection, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 M00 SG

Part No.: 41.052.4041.4

RST® MICRO device connector M16, IP66/68(3m;2h)/69, male, 5-pole, ~50/120 V, 8 A, with contact seal, slate grey

System RST® MICRO - Device connector, male, 5-pole, turned contacts with screw connection, with contact seal, fastening screw from inside, mechanical/color coding slate grey.

Rated voltage: 250/400 V

Rated current: 8 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/68(3m;2h)/69

Rated cross-section (single-stranded): 0.2 - 1.0 mm²

Rated cross-section (fine-stranded): 0.22 - 1.0 mm²

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Connection per pole: 1

Marking: 1, 2, 3, 4, 5

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08I5S S1 M00 SG

Part No.: 4L.052.4041.4

  - Accessories

RST® MICRO cover piece IP66/68(3m;2h)/69 for female, 4-/5-pole, captive against loss, black

System RST® MICRO - Cover piece IP66/68(3m;2h)/69 for female parts RST08, 4-/5-pole, with mounting strap for snapping on, black.

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08 S VS SW

Part No.: 06.564.1553.1

RST® MICRO cover piece IP66/68(3m;2h)/69 for male, 4-/5-pole, captive against loss, black

System RST® MICRO - Cover piece IP66/68(3m;2h)/69 for male parts RST08, 4-/5-pole, with mounting strap for snapping on, black.

Housing material: Polyamide, halogen-free, V0, f1

Seal material: NBR

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST08 B VS SW

Part No.: 06.564.1653.1

RST® MICRO Pin wrench for screw terminals

System RST® MICRO - Pin wrench for RST® MICRO screw terminals.

Brand: Wieland Electric GmbH

Type: RST08 S

Part No.: 06.502.6100.0

RST® MICRO Hexagonal wrench for screw terminals

System RST® MICRO - Hexagonal wrench for RST® MICRO screw terminals.

Brand: Wieland Electric GmbH

Type: RST08 S

Part No.: 06.502.6300.0

06.502.6100.0	24, 46, 62, 78

06.502.6300.0	24, 46, 62, 78

06.563.1553.1	62, 78

06.563.5053.1	24, 46

06.563.5153.1	24, 46

06.564.1653.1	62, 78

41.021.3041.4	4

41.021.3041.8	4

41.021.3043.1	2

41.021.3043.9	3

41.021.4041.4	11

41.021.4041.8	12

41.021.4043.1	10

41.021.4043.9	11

41.022.3041.4	7

41.022.3041.8	8

41.022.3043.1	6

41.022.3043.9	7

41.022.4041.4	15

41.022.4041.8	16

41.022.4043.1	14

41.022.4043.9	15

41.02L.3043.1	2

41.031.3041.4	27

41.031.3041.8	27

41.031.3043.9	26

41.031.3053.1	25

41.031.4041.4	34

41.031.4041.8	35

41.031.4043.9	34

41.031.4053.1	33

41.032.3041.4	30

41.032.3041.8	31

41.032.3043.9	30

41.032.3053.1	29

41.032.4041.4	38

41.032.4041.8	39

41.032.4043.9	38

41.032.4053.1	37

41.041.3041.4	49

41.041.3041.8	49

41.041.3043.9	48

41.041.3053.1	47

41.041.4041.4	57

41.041.4041.8	56

41.041.4043.9	56

41.041.4053.1	55

41.042.3041.4	53

41.042.3041.8	52

41.042.3043.9	52

41.042.3053.1	51

41.042.4041.4	61

41.042.4041.8	60

41.042.4043.9	60

41.042.4053.1	59

41.051.3041.4	65

41.051.3041.8	65

41.051.3043.9	64

41.051.3053.1	63

41.051.4041.4	73

41.051.4041.8	72

41.051.4043.9	72

41.051.4053.1	71

41.052.3041.4	69

41.052.3041.8	68

41.052.3043.9	68

41.052.3053.1	67

41.052.4041.4	77

41.052.4041.8	76

41.052.4043.9	76

41.052.4053.1	75

41.427.x030.1	20

41.427.x030.9	20

41.427.x032.4	20

41.427.x032.8	21

41.427.x033.1	21

41.427.x033.9	21

41.427.x034.1	22

41.427.x034.9	23

41.427.x037.4	22

41.427.x037.8	22

41.427.x038.4	23

41.427.x038.8	23

41.437.x030.1	43

41.437.x032.8	43

41.437.x033.1	44

41.437.x034.1	45

41.437.x037.8	44

41.437.x038.8	45

46.020.0141.4	19

46.020.0143.1	18

46.020.0143.9	19

46.020.0241.8	19

46.020.1141.4	18

46.020.1143.1	17

46.020.1143.9	17

46.020.1241.8	18

46.030.0141.4	42

46.030.0143.1	41

46.030.0143.9	42

46.030.0341.8	42

46.030.1141.4	41

46.030.1143.1	40

46.030.1143.9	40

46.030.1341.8	41

4L.021.3041.4	4

4L.021.3041.8	5

4L.021.3043.9	3

4L.021.4041.4	12

4L.021.4041.8	13

4L.021.4043.1	10

4L.021.4043.9	11

4L.022.3041.4	8

4L.022.3041.8	9

4L.022.3043.1	6

4L.022.3043.9	7

4L.022.4041.4	16

4L.022.4041.8	16

4L.022.4043.1	14

4L.022.4043.9	15

4L.031.3041.4	27

4L.031.3041.8	28

4L.031.3043.9	26

4L.031.3053.1	25

4L.031.4041.4	35

4L.031.4041.8	36

4L.031.4043.9	34

4L.031.4053.1	33

4L.032.3041.4	31

4L.032.3041.8	32

4L.032.3043.9	30

4L.032.3053.1	29

4L.032.4041.4	39

4L.032.4041.8	39

4L.032.4043.9	38

4L.032.4053.1	37

4L.041.3041.4	50

4L.041.3041.8	49

4L.041.3043.9	48

4L.041.3053.1	47

4L.041.4041.4	58

4L.041.4041.8	57

4L.041.4043.9	56

4L.041.4053.1	55

4L.042.3041.4	54

4L.042.3041.8	53

4L.042.3043.9	52

4L.042.3053.1	51

4L.042.4041.4	61

4L.042.4041.8	61

4L.042.4043.9	60

4L.042.4053.1	59

4L.051.3041.4	66

4L.051.3041.8	65

4L.051.3043.9	64

4L.051.3053.1	63

4L.051.4041.4	74

4L.051.4041.8	73

4L.051.4043.9	72

4L.051.4053.1	71

4L.052.3041.4	70

4L.052.3041.8	69

4L.052.3043.9	68

4L.052.3053.1	67

4L.052.4041.4	77

4L.052.4041.8	77

4L.052.4043.9	76

4L.052.4053.1	75

4L.437.x030.1	43

4L.437.x033.1	44

4L.437.x034.1	45