1	RST® POWER - Circular connector system IP66/67/69K - 4-pole - RST50i4	2

1.1	Connector	2

1.1.1	Cable diameter 15-25 mm	2

1.1.1.1	Female with screw connection	2

1.1.1.3	Male with screw connection	3

1.1.2	Cable diameter 20-32 mm	4

1.1.2.1	Female with screw connection	4

1.1.2.3	Male with screw connection	5

1.2	Device connector	6

1.2.1	M32	6

1.2.1.1	Female with screw connection	6

1.2.1.3	Male with screw connection	7

1.3	Accessories	8

2	RST® POWER - Circular connector system IP66/67/69K - 5-pole - RST50i5	9

2.1	Connector	9

2.1.1	Cable diameter 15-25 mm	9

2.1.1.1	Female with screw connection	9

2.1.1.3	Male with screw connection	10

2.1.2	Cable diameter 20-32 mm	11

2.1.2.1	Female with screw connection	11

2.1.2.3	Male with screw connection	12

2.2	Device connector	13

2.2.1	M32	13

2.2.1.1	Female with screw connection	13

2.2.1.3	Male with screw connection	14

2.3	Accessories	15

- RST® POWER - Circular connector system IP66/67/69K - 4-pole - RST50i4

  - Connector

    - Cable diameter 15-25 mm

      - Female with screw connection

RST® circular connector, IP66/67/69K, female, 4-pole, 250/400 V, 50 A

System RST® POWER - Circular connector with type of protection IP66/67/69K, female, 4-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section (solid/stranded): 4.0 - 6.0 mm²

Terminal cross-section (flexible wires): 4.0 - 16.0 mm²

Cable diameter: 15 - 25 mm

Cable gland: M32

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I4S B1 ZR5 S SW

Part No.: 97.041.4053.1

      - Male with screw connection

RST® circular connector, IP66/67/69K, male, 4-pole, 250/400 V, 50 A

System RST® POWER - Circular connector with type of protection IP66/67/69K, male, 4-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section (solid/stranded): 4.0 - 6.0 mm²

Terminal cross-section (flexible wires): 4.0 - 16.0 mm²

Cable diameter: 15 - 25 mm

Cable gland: M32

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I4S S1 ZR5 S SW

Part No.: 97.042.4053.1

    - Cable diameter 20-32 mm

      - Female with screw connection

RST® circular connector, IP66/67/69K, female, 4-pole, 250/400 V, 50 A

System RST® POWER - Circular connector with type of protection IP66/67/69K, female, 4-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section (solid/stranded): 4.0 - 6.0 mm²

Terminal cross-section (flexible wires): 4.0 - 16.0 mm²

Cable diameter: 20 - 32 mm

Cable gland: M40

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I4S B1 ZR6 S SW

Part No.: 97.041.4253.1

      - Male with screw connection

RST® circular connector, IP66/67/69K, male, 4-pole, 250/400 V, 50 A

System RST® POWER - Circular connector with type of protection IP66/67/69K, male, 4-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section (solid/stranded): 4.0 - 6.0 mm²

Terminal cross-section (flexible wires): 4.0 - 16.0 mm²

Cable diameter: 20 - 32 mm

Cable gland: M40

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I4S S1 ZR6 S SW

Part No.: 97.042.4253.1

  - Device connector

    - M32

      - Female with screw connection

RST® Device connector, IP66/67/69K, female, 4-pole, 250/400 V, 50 A, fixed in position

System RST® POWER - Device connector with type of protection IP66/67/69K, female, 4-pole, turned contacts with screw connection fixed in position, compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section: 4.0 - 16.0 mm²

Cable gland: M32

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I4S B1 ZM02V SW

Part No.: 97.041.5553.1

RST® Device connector, IP66/67/69K, female, 4-pole, 250/400 V, 50 A

System RST® POWER - Device connector with type of protection IP66/67/69K, female, 4-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section: 4.0 - 16.0 mm²

Cable gland: M32

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I4S B1 M02V SW

Part No.: 97.041.5053.1

      - Male with screw connection

RST® Device connector, IP66/67/69K, male, 4-pole, 250/400 V, 50 A, fixed in position

System RST® POWER - Device connector with type of protection IP66/67/69K, male, 4-pole, turned contacts with screw connection fixed in position, compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section: 4.0 - 16.0 mm²

Cable gland: M32

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I4S S1 ZM02V SW

Part No.: 97.042.5553.1

RST® Device connector, IP66/67/69K, male, 4-pole, 250/400 V, 50 A

System RST® POWER - Device connector with type of protection IP66/67/69K, male, 4-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section: 4.0 - 16.0 mm²

Cable gland: M32

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I4S S1 M02V SW

Part No.: 97.042.5053.1

  - Accessories

RST® cover IP66/67/69K for female/male, 4-5-pole, black

System RST® POWER - Cover IP66/67/69K for female/male parts RST50i4 and RST50i5, black.

Housing material: PA66, V2

Sealing material: NBR

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50 VS SW

Part No.: Z5.567.5653.0

RST® adapter ring, 40 mm

System RST® POWER - Adapter ring for fixing the device connector inside 40 mm knock-outs, black.

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50

Part No.: 05.568.1853.0

- RST® POWER - Circular connector system IP66/67/69K - 5-pole - RST50i5

  - Connector

    - Cable diameter 15-25 mm

      - Female with screw connection

RST® circular connector, IP66/67/69K, female, 5-pole, 250/400 V, 50 A

System RST® POWER - Circular connector with type of protection IP66/67/69K, female, 5-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section (solid/stranded): 4.0 - 6.0 mm²

Terminal cross-section (flexible wires): 4.0 - 16.0 mm²

Cable diameter: 15 - 25 mm

Cable gland: M32

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I5S B1 ZR5 S SW

Part No.: 97.051.4053.1

      - Male with screw connection

RST® circular connector, IP66/67/69K, male, 5-pole, 250/400 V, 50 A

System RST® POWER - Circular connector with type of protection IP66/67/69K, male, 5-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section (solid/stranded): 4.0 - 6.0 mm²

Terminal cross-section (flexible wires): 4.0 - 16.0 mm²

Cable diameter: 15 - 25 mm

Cable gland: M32

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I5S S1 ZR5 S SW

Part No.: 97.052.4053.1

    - Cable diameter 20-32 mm

      - Female with screw connection

RST® circular connector, IP66/67/69K, female, 5-pole, 250/400 V, 50 A

System RST® POWER - Circular connector with type of protection IP66/67/69K, female, 5-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section (solid/stranded): 4.0 - 6.0 mm²

Terminal cross-section (flexible wires): 4.0 - 16.0 mm²

Cable diameter: 20 - 32 mm

Cable gland: M40

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I5S B1 ZR6 S SW

Part No.: 97.051.4253.1

      - Male with screw connection

RST® circular connector, IP66/67/69K, male, 5-pole, 250/400 V, 50 A

System RST® POWER - Circular connector with type of protection IP66/67/69K, male, 5-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section (solid/stranded): 4.0 - 6.0 mm²

Terminal cross-section (flexible wires): 4.0 - 16.0 mm²

Cable diameter: 20 - 32 mm

Cable gland: M40

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I5S S1 ZR6 S SW

Part No.: 97.052.4253.1

  - Device connector

    - M32

      - Female with screw connection

RST® Device connector, IP66/67/69K, female, 5-pole, 250/400 V, 50 A, fixed in position

System RST® POWER - Device connector with type of protection IP66/67/69K, female, 5-pole, turned contacts with screw connection fixed in position, compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section: 4.0 - 16.0 mm²

Cable gland: M32

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I5S B1 ZM02V SW

Part No.: 97.051.5553.1

RST® Device connector, IP66/67/69K, female, 5-pole, 250/400 V, 50 A

System RST® POWER - Device connector with type of protection IP66/67/69K, female, 5-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section: 4.0 - 16.0 mm²

Cable gland: M32

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I5S B1 M02V SW

Part No.: 97.051.5053.1

      - Male with screw connection

RST® Device connector, IP66/67/69K, male, 5-pole, 250/400 V, 50 A, fixed in position

System RST® POWER - Device connector with type of protection IP66/67/69K, male, 5-pole, turned contacts with screw connection fixed in position, compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section: 4.0 - 16.0 mm²

Cable gland: M32

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I5S S1 ZM02V SW

Part No.: 97.052.5553.1

RST® Device connector, IP66/67/69K, male, 5-pole, 250/400 V, 50 A

System RST® POWER - Device connector with type of protection IP66/67/69K, male, 5-pole, turned contacts with screw connection compact housing with strain relief, field assembly, mechanical/color coding black.

Rated voltage: 250/400 V

Rated current: 50 A

Rated impulse voltage: 4 kV

Type of protection: up to IP66/67/69K

Terminal cross-section: 4.0 - 16.0 mm²

Cable gland: M32

Housing material: PA66, V2

Sealing material: NBR

Connections per pole: 1

Marking: PE, 1, 2, 3, N

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50I5S S1 M02V SW

Part No.: 97.052.5053.1

  - Accessories

RST® Cover IP66/67/69K for female/male, 4-5-pole, black

System RST® POWER - Cover IP66/67/69K for female/male parts RST50i4 and RST50i5, black.

Housing material: PA66, V2

Sealing material: NBR

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50 VS SW

Part No.: Z5.567.5653.0

RST® adapter ring, 40 mm

System RST® POWER - Adapter ring for fixing the device connector inside 40 mm knock-outs, black.

Halogen-free: yes

Brand: Wieland Electric GmbH

Type: RST50

Part No.: 05.568.1853.0

02.126.0621.8	14, 28

02.126.0721.8	14, 28

02.126.0821.8	14, 28

05.502.2300.0	15, 29

05.545.2821.8	14, 28

05.545.2921.8	15, 29

05.545.3021.8	15, 29

05.568.1853.0	15, 29

95.101.0800.0	15, 29

97.041.1053.1	11

97.041.4053.1	2

97.041.4253.1	6

97.041.5053.1	10

97.041.5553.1	10

97.042.1053.1	13

97.042.4053.1	4

97.042.4253.1	8

97.042.5053.1	12

97.042.5553.1	12

97.051.1053.1	25

97.051.4053.1	16

97.051.4253.1	20

97.051.5053.1	24

97.051.5553.1	24

97.052.1053.1	27

97.052.4053.1	18

97.052.4253.1	22

97.052.5053.1	26

97.052.5553.1	26

97.141.0053.1	3

97.141.0253.1	7

97.141.1553.1	11

97.142.0053.1	5

97.142.0253.1	9

97.142.1553.1	13

97.151.0053.1	17

97.151.0253.1	21

97.151.1553.1	25

97.152.0053.1	19

97.152.0253.1	23

97.152.1553.1	27

99.628.0000.0	29

Z5.567.5653.0	14, 28