1	gesis® MICRO - Installation connector system	2

1.1	2-pole - GST08i2	2

1.1.1	Distribution blocks	2

1.1.2	Cables	5

1.1.2.1	Cable assemblies, 0.5 mm², PVC, heat resistant, containing halogen	5

1.1.2.2	Cable assemblies, 0.75 mm², PVC, containing halogen	8

1.1.2.3	Cable assemblies, 0.75 mm², PVC, heat resistant, containing halogen	13

1.1.2.4	Cable assemblies, 0.75 mm², halogen free	20

1.1.3	Accessories	22

1.2	3-pole - GST08i3	23

1.2.1	Distribution blocks	23

1.2.2	Cables	24

1.2.2.1	Cable assemblies, 0.75 mm², containing halogen	24

1.2.3	Accessories	26

- gesis® MICRO - Installation connector system

  - 2-pole - GST08i2

    - Distribution blocks

gesis® distribution block, 2-pole, 1 input, 2 outputs, 250 V, 2.5 A, white

System gesis® MICRO - Distribution block, h-shaped distribution block, 2-pole, 1 input, 2 outputs, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 2.5 A

Input: 1

Outputs: 2

Brand: Wieland Electric GmbH

Type: GST08i2 V 2P WS01

Part No.: 90.030.0153.0

gesis® distribution block, 2-pole, 1 input, 2 outputs, SELV, 6 A, light blue

System gesis® MICRO - Distribution block, h-shaped distribution block, 2-pole, 1 input, 2 outputs, mechanical coding code 2, color coding light blue.

Rated voltage: SELV

Rated current: 6 A

Input: 1

Outputs: 2

Brand: Wieland Electric GmbH

Type: GST08i2 V 2P BL02

Part No.: 90.000.0153.0

gesis® distribution block, 2-pole, 1 input, 2 outputs, SELV, 6 A, green

System gesis® MICRO - Distribution block, h-shaped distribution block, 2-pole, 1 input, 2 outputs, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Input: 1

Outputs: 2

Brand: Wieland Electric GmbH

Type: GST08i2 V 2P GN03

Part No.: 90.010.0153.0

gesis® distribution block, 2-pole, 1 input, 2 outputs, SELV DC, 6 A, yellow

System gesis® MICRO - Distribution block, h-shaped distribution block, 2-pole, 1 input, 2 outputs, mechanical coding code 5, color coding yellow.

Rated voltage: SELV DC

Rated current: 6 A

Input: 1

Outputs: 2

Brand: Wieland Electric GmbH

Type: GST08i2 V 2P BL02

Part No.: 90.020.0153.0

gesis® cable with 6-fold distribution block, male, 2x 0.75 mm², 250 V, 2.5 A, length: 0.5 m, white

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, 6-fold female GST08i2, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 2.5 A

Cable type: H03V2V2H2-F

Cable color: white

Input: 1

Outputs: 6

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 VS 6P 75 T 05WS01

Part No.: 90.230.0526.2

gesis® cable with 6-fold distribution block, free end, 2x 0.75 mm², SELV, 6 A, length: 3 m, green

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², 6-fold female GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Input: 1

Outputs: 6

Halogen-free: no

Length: 3 m

Brand: Wieland Electric GmbH

Type: GST08i2 VF 6P 75 T 30GN03

Part No.: 90.210.3026.2

gesis® cable with 6-fold distribution block, free end, 2x 0.75 mm², SELV, 6 A, length: 0.2 m, green

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², 6-fold female GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Input: 1

Outputs: 6

Halogen-free: no

Length: 0.2 m

Brand: Wieland Electric GmbH

Type: GST08i2 VF 6P 75 T 02GN03

Part No.: 99.419.0000.8

gesis® cable with 6-fold distribution block, free end, 2x 0.75 mm², SELV, 6 A, length: 0.2 m, light blue

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², 6-fold female GST08i2, free end, mechanical coding code 3, color coding light blue.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Input: 1

Outputs: 6

Halogen-free: no

Length: 0.2 m

Brand: Wieland Electric GmbH

Type: GST08i2 VF 6P 75 T 02BL02

Part No.: 99.413.0000.8

    - Cables

      - Cable assemblies, 0.5 mm², PVC, heat resistant, containing halogen

gesis® extension cable, female, male, 2x 0.5 mm², SELV, 6 A, length: 0.5 m, green

System gesis® MICRO - assembled LIYZ extension cable, 2-pole, 0.5 mm², male GST08i2, female GST08i2, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: LIYZ

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 05T 05GN03

Part No.: 90.215.0530.2

gesis® extension cable, female, male, 2x 0.5 mm², SELV, 6 A, length: 1 m, green

System gesis® MICRO - assembled LIYZ extension cable, 2-pole, 0.5 mm², male GST08i2, female GST08i2, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: LIYZ

Cable color: white

Halogen-free: no

Length: 1 m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 05T 10GN03

Part No.: 90.215.1030.2

gesis® extension cable, female, male, 2x 0.5 mm², SELV, 6 A, length: 1.5 m, green

System gesis® MICRO - assembled LIYZ extension cable, 2-pole, 0.5 mm², male GST08i2, female GST08i2, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: LIYZ

Cable color: white

Halogen-free: no

Length: 1.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 05T 15GN03

Part No.: 90.215.1530.2

gesis® connection cable, female, free end, 2x 0.5 mm², SELV, 6 A, length: 0.5 m, green

System gesis® MICRO - assembled LIYZ extension cable, 2-pole, 0.5 mm², female GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: LIYZ

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 05T 05GN03

Part No.: 90.215.0533.2

gesis® connection cable, female, free end, 2x 0.5 mm², SELV, 6 A, length: 1 m, green

System gesis® MICRO - assembled LIYZ extension cable, 2-pole, 0.5 mm², female GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: LIYZ

Cable color: white

Halogen-free: no

Length: 1 m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 05T 10GN03

Part No.: 90.215.1033.2

gesis® connection cable, female, free end, 2x 0.5 mm², SELV, 6 A, length: 1.5 m, green

System gesis® MICRO - assembled LIYZ extension cable, 2-pole, 0.5 mm², female GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: LIYZ

Cable color: white

Halogen-free: no

Length: 1.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 05T 15GN03

Part No.: 90.215.1533.2

gesis® connection cable, male, free end, 2x 0.5 mm², SELV, 6 A, length: 0.5 m, green

System gesis® MICRO - assembled LIYZ extension cable, 2-pole, 0.5 mm², male GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: LIYZ

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 05T 05GN03

Part No.: 90.215.0534.2

gesis® connection cable, male, free end, 2x 0.5 mm², SELV, 6 A, length: 1 m, green

System gesis® MICRO - assembled LIYZ extension cable, 2-pole, 0.5 mm², male GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: LIYZ

Cable color: white

Halogen-free: no

Length: 1 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 05T 10GN03

Part No.: 90.215.1034.2

gesis® connection cable, male, free end, 2x 0.5 mm², SELV, 6 A, length: 1.5 m, green

System gesis® MICRO - assembled LIYZ extension cable, 2-pole, 0.5 mm², male GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: LIYZ

Cable color: white

Halogen-free: no

Length: 1.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 05T 15GN03

Part No.: 90.215.1534.2

      - Cable assemblies, 0.75 mm², PVC, containing halogen

gesis® extension cable, female, male, 2x 0.75 mm², 250 V, 2.5 A, length: x m, white

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², male GST08i2, female GST08i2, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 2.5 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 75P x0WS01

Part No.: 90.230.x010.2 (x corresponds to length in m)

gesis® extension cable, female, male, 2x 0.75 mm², SELV, 6 A, length: 2.5 m, green

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², male GST08i2, female GST08i2, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: 2.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 75P 25GN03

Part No.: 90.210.2510.2

gesis® extension cable, female, male, 2x 0.75 mm², SELV, 6 A, length: x m, green

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², male GST08i2, female GST08i2, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 75P x0GN03

Part No.: 90.210.x010.2 (x corresponds to length in m)

gesis® extension cable, female, male, 2x 0.75 mm², SELV, 6 A, length: x m, light blue

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², male GST08i2, female GST08i2, mechanical coding code 3, color coding light blue.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 75P x0BL02

Part No.: 90.200.x010.2 (x corresponds to length in m)

gesis® extension cable, female, male, 2x 0.75 mm², SELV DC, 6 A, length: x m, yellow

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², male GST08i2, female GST08i2, mechanical coding code 3, color coding green.

Rated voltage: SELV DC

Rated current: 6 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 75P x0GE05

Part No.: 90.220.x010.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 2x 0.75 mm², 250 V, 2.5 A, length: 0.5 m, white

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², female GST08i2, free end, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 2.5 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 75P 05WS01

Part No.: 90.230.0513.2

gesis® connection cable, female, free end, 2x 0.75 mm², SELV, 6 A, length: 0.3 m, green

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², female GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: 0.3 m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 75P 03GN03

Part No.: 90.210.0313.2

gesis® connection cable, female, free end, 2x 0.75 mm², SELV, 6 A, length: 0.5 m, green

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², female GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 75P 05GN03

Part No.: 90.210.0513.2

gesis® connection cable, female, free end, 2x 0.75 mm², SELV, 6 A, length: x m, green

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², female GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 75P x0GN03

Part No.: 90.210.x013.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 2x 0.75 mm², SELV, 6 A, length: 0.5 m, light blue

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², female GST08i2, free end, mechanical coding code 3, color coding light blue.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 75P 05BL02

Part No.: 90.200.0513.2

gesis® connection cable, male, free end, 2x 0.75 mm², 250 V, 2.5 A, length: 0.5 m, white

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 2.5 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75P 05WS01

Part No.: 90.230.0514.2

gesis® connection cable, male, free end, 2x 0.75 mm², 250 V, 2.5 A, length: 2 m, white

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 2.5 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: 2 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75P 20WS01

Part No.: 90.230.2014.2

gesis® connection cable, male, free end, 2x 0.75 mm², SELV, 6 A, length: 0.5 m, green

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75P 05GN03

Part No.: 90.210.0514.2

gesis® connection cable, male, free end, 2x 0.75 mm², SELV, 6 A, length: 2.5 m, green

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: 2.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75P 25GN03

Part No.: 90.210.2514.2

gesis® connection cable, male, free end, 2x 0.75 mm², SELV, 6 A, length: x m, green

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75P x0GN03

Part No.: 90.210.x014.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 2x 0.75 mm², SELV, 6 A, length: 0.5 m, light blue

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 3, color coding light blue.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75P 05BL02

Part No.: 90.200.0514.2

gesis® connection cable, male, free end, 2x 0.75 mm², SELV DC, 6 A, length: 0.5 m, yellow

System gesis® MICRO - assembled H03VVH2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 5, color coding yellow.

Rated voltage: SELV DC

Rated current: 6 A

Cable type: H03VVH2-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75P 05GE05

Part No.: 90.220.0514.2

      - Cable assemblies, 0.75 mm², PVC, heat resistant, containing halogen

gesis® extension cable, female, male, 2x 0.75 mm², 250 V, 2.5 A, length: x m, white

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, female GST08i2, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 2.5 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 75T x0WS01

Part No.: 90.230.x020.2 (x corresponds to length in m)

gesis® extension cable, female, male, 2x 0.75 mm², SELV, 6 A, length: 2.5 m, green

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, female GST08i2, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: 2.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 75T 25GN03

Part No.: 90.210.2520.2

gesis® extension cable, female, male, 2x 0.75 mm², SELV, 6 A, length: x m, green

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, female GST08i2, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 75T x0GN03

Part No.: 90.210.x020.2 (x corresponds to length in m)

gesis® extension cable, female, male, 2x 0.75 mm², SELV, 6 A, length: x m, light blue

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, female GST08i2, mechanical coding code 3, color coding light blue.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 75T x0BL02

Part No.: 90.200.x020.2 (x corresponds to length in m)

gesis® extension cable, female, male, 2x 0.75 mm², SELV DC, 6 A, length: x m, yellow

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, female GST08i2, mechanical coding code 3, color coding green.

Rated voltage: SELV DC

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 75T x0GE05

Part No.: 90.220.x020.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 2x 0.75 mm², 250 V, 2.5 A, length: 0.5 m, white

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², female GST08i2, free end, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 2.5 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 75T 05WS01

Part No.: 90.230.0523.2

gesis® connection cable, female, free end, 2x 0.75 mm², SELV, 6 A, length: 0.15 m, green

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², female GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: 0.15 m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 75T 02GN03

Part No.: 99.415.0000.8

gesis® connection cable, female, free end, 2x 0.75 mm², SELV, 6 A, length: 0.3 m, green

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², female GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: 0.3 m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 75T 03GN03

Part No.: 90.210.0323.2

gesis® connection cable, female, free end, 2x 0.75 mm², SELV, 6 A, length: 0.5 m, green

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², female GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 75T 05GN03

Part No.: 90.210.0523.2

gesis® connection cable, female, free end, 2x 0.75 mm², SELV, 6 A, length: x m, green

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², female GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 75T x0GN03

Part No.: 90.210.x023.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 2x 0.75 mm², SELV, 6 A, length: 0.5 m, light blue

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², female GST08i2, free end, mechanical coding code 3, color coding light blue.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 75T 0.5BL02

Part No.: 90.200.0.523.2

gesis® connection cable, female, free end, 2x 0.75 mm², SELV, 6 A, length: x m, light blue

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², female GST08i2, free end, mechanical coding code 3, color coding light blue.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 75T x0BL02

Part No.: 90.200.x023.2 (x corresponds to length in m)

gesis® connection cable, female, free end, 2x 0.75 mm², SELV DC, 6 A, length: 0.5 m, yellow

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², female GST08i2, free end, mechanical coding code 5, color coding yellow.

Rated voltage: SELV DC

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 75T 05GN03

Part No.: 90.220.0523.2

gesis® connection cable, male, free end, 2x 0.75 mm², 250 V, 2.5 A, length: 0.5 m, white

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 2.5 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75T 05WS01

Part No.: 90.230.0524.2

gesis® connection cable, male, free end, 2x 0.75 mm², 250 V, 2.5 A, length: x m, white

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 2.5 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75T x0WS01

Part No.: 90.230.x024.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 2x 0.75 mm², SELV, 6 A, length: 0.15 m, green

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: 0.15 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75T 02GN03

Part No.: 99.428.0000.8

gesis® connection cable, male, free end, 2x 0.75 mm², SELV, 6 A, length: 0.5 m, green

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75T 05GN03

Part No.: 90.210.0524.2

gesis® connection cable, male, free end, 2x 0.75 mm², SELV, 6 A, length: 2.5 m, green

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: 2.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75T 25GN03

Part No.: 90.210.2524.2

gesis® connection cable, male, free end, 2x 0.75 mm², SELV, 6 A, length: x m, green

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75T x0GN03

Part No.: 90.210.x024.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 2x 0.75 mm², SELV, 6 A, length: 0.5 m, light blue

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 3, color coding light blue.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75T 05BL02

Part No.: 90.200.0524.2

gesis® connection cable, male, free end, 2x 0.75 mm², SELV, 6 A, length: x m, light blue

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 3, color coding light blue.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: x m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75T x0BL02

Part No.: 90.200.x024.2 (x corresponds to length in m)

gesis® connection cable, male, free end, 2x 0.75 mm², SELV DC, 6 A, length: 0.5 m, yellow

System gesis® MICRO - assembled H03V2V2H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 5, color coding yellow.

Rated voltage: SELV DC

Rated current: 6 A

Cable type: H03V2V2H2-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75T 05GE05

Part No.: 90.220.0524.2

      - Cable assemblies, 0.75 mm², halogen free

gesis® extension cable, female, male, 2x 0.75 mm², 250 V, 2.5 A, length: 1 m, white

System gesis® MICRO - assembled H03Z1Z1H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, female GST08i2, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 2.5 A

Cable type: H03Z1Z1H2-F

Cable color: white

Halogen-free: yes

Length: 1 m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 75H 10WS01

Part No.: 90.230.1040.2

gesis® extension cable, female, male, 2x 0.75 mm², 250 V, 2.5 A, length: 3 m, white

System gesis® MICRO - assembled H03Z1Z1H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, female GST08i2, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 2.5 A

Cable type: H03Z1Z1H2-F

Cable color: white

Halogen-free: yes

Length: 3 m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 75H 30WS01

Part No.: 90.230.3040.2

gesis® extension cable, female, male, 2x 0.75 mm², SELV, 6 A, length: 3 m, green

System gesis® MICRO - assembled H03Z1Z1H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, female GST08i2, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03Z1Z1H2-F

Cable color: white

Halogen-free: yes

Length: 3 m

Brand: Wieland Electric GmbH

Type: GST08i2 BS 75H 30GN03

Part No.: 90.210.3040.2

gesis® connection cable, female, free end, 2x 0.75 mm², SELV, 6 A, length: 0.5 m, green

System gesis® MICRO - assembled H03Z1Z1H2-F extension cable, 2-pole, 0.75 mm², female GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03Z1Z1H2-F

Cable color: white

Halogen-free: yes

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 B- 75H 05GN03

Part No.: 90.210.0543.2

gesis® connection cable, male, free end, 2x 0.75 mm², 250 V, 2.5 A, length: 0.5 m, white

System gesis® MICRO - assembled H03Z1Z1H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 2.5 A

Cable type: H03Z1Z1H2-F

Cable color: white

Halogen-free: yes

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75H 05WS01

Part No.: 90.230.0544.2

gesis® connection cable, male, free end, 2x 0.75 mm², SELV, 6 A, length: 0.5 m, green

System gesis® MICRO - assembled H03Z1Z1H2-F extension cable, 2-pole, 0.75 mm², male GST08i2, free end, mechanical coding code 3, color coding green.

Rated voltage: SELV

Rated current: 6 A

Cable type: H03Z1Z1H2-F

Cable color: white

Halogen-free: yes

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i2 -S 75 05GN03

Part No.: 90.210.0544.2

H

    - Accessories

gesis® housing installation frame, white

System gesis® MICRO - Housing installation frame for 2-pole female connector, incl. fixation screw, white.

Brand: Wieland Electric GmbH

Type: GST08i2 G

Part No.: 99.450.0000.8

gesis® cover cap

System gesis® MICRO - Cover cap for GST08i2 female connectors, white.

Brand: Wieland Electric GmbH

Type: GST08i2 CC

Part No.: 06.562.3100.0

gesis® sealing grommet

System gesis® MICRO - Sealing grommet IP44, transparent.

Brand: Wieland Electric GmbH

Type: GST08i2 SG

Part No.: 06.562.3200.0

  - 3-pole - GST08i3

    - Distribution blocks

gesis® cable with 6-fold distribution block, male, 3x 0.75 mm², 250 V, 6 A, length: 0.5 m, white

System gesis® MICRO - assembled H03VV-F extension cable with protective ground, 3-pole, 0.75 mm², male GST08i3, 6-fold female GST08i3, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 6 A

Cable type: H03VV-F

Cable color: white

Input: 1

Outputs: 6

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i3 VS 6P 75 P 05WS01

Part No.: 90.240.0506.2

    - Cables

      - Cable assemblies, 0.75 mm², containing halogen

gesis® extension cable, female, male, 3x 0.75 mm², 250 V, 6 A, length: 1 m, white

System gesis® MICRO - assembled H03VV-F extension cable with protective ground, 3-pole, 0.75 mm², male GST08i3, female GST08i3, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 6 A

Cable type: H03VV-F

Cable color: white

Halogen-free: no

Length: 1 m

Brand: Wieland Electric GmbH

Type: GST08i3 BS 75 T 10WS01

Part No.: 90.240.1000.2

gesis® extension cable, female, male, 3x 0.75 mm², 250 V, 6 A, length: 3 m, white

System gesis® MICRO - assembled H03VV-F extension cable with protective ground, 3-pole, 0.75 mm², male GST08i3, female GST08i3, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 6 A

Cable type: H03VV-F

Cable color: white

Halogen-free: no

Length: 3 m

Brand: Wieland Electric GmbH

Type: GST08i3 BS 75 T 30WS01

Part No.: 90.240.3000.2

gesis® connection cable, female, free end, 3x 0.75 mm², 250 V, 6 A, length: 0.5 m, white

System gesis® MICRO - assembled H03VV-F extension cable with protective ground, 3-pole, 0.75 mm², female GST08i3, free end, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 6 A

Cable type: H03VV-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i3 B- 75 T 05WS01

Part No.: 90.240.0503.2

gesis® connection cable, male, free end, 3x 0.75 mm², 250 V, 6 A, length: 0.5 m, white

System gesis® MICRO - assembled H03VV-F extension cable with protective ground, 3-pole, 0.75 mm², male GST08i3, free end, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current: 6 A

Cable type: H03VV-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i3 -S 75 T 05WS01

Part No.: 90.240.0504.2

gesis® connection cable, female, male, 3x 0.75 mm², 250 V, 6 A, length: 0.5 m, white

System gesis® MICRO - assembled H03VV-F extension cable with protective ground, 3-pole, 0.75 mm², male GST15i3, female GST08i3, mechanical coding code 1, color coding white.

Rated voltage: 250 V

Rated current:6 A

Cable type: H03VV-F

Cable color: white

Halogen-free: no

Length: 0.5 m

Brand: Wieland Electric GmbH

Type: GST08i3 BB 75 P 05WS01

Part No.: 90.240.0505.2

    - Accessories

gesis® housing installation frame, white

System gesis® MICRO - Housing installation frame for 3-pole female, incl. fixation screw, white.

Brand: Wieland Electric GmbH

Type: GST08i3 G

Part No.: 99.451.0000.8

06.562.3100.0	22

06.562.3200.0	22

90.000.0153.0	2

90.010.0153.0	2

90.020.0153.0	3

90.030.0153.0	2

90.200.0513.2	10

90.200.0514.2	12

90.200.0523.2	16

90.200.0524.2	18

90.200.x010.2	9

90.200.x020.2	14

90.200.x023.2	16

90.200.x024.2	18

90.210.0313.2	9

90.210.0323.2	15

90.210.0513.2	10

90.210.0514.2	11

90.210.0523.2	15

90.210.0524.2	17

90.210.0543.2	21

90.210.0544.2	21

90.210.2510.2	8

90.210.2514.2	11

90.210.2520.2	13

90.210.2524.2	17

90.210.3026.2	3

90.210.3040.2	20

90.210.x010.2	8

90.210.x013.2	10

90.210.x014.2	12

90.210.x020.2	13

90.210.x023.2	15

90.210.x024.2	18

90.215.0530.2	5

90.215.0533.2	6

90.215.0534.2	6

90.215.1030.2	5

90.215.1033.2	6

90.215.1034.2	7

90.215.1530.2	5

90.215.1533.2	6

90.215.1534.2	7

90.220.0514.2	12

90.220.0523.2	16

90.220.0524.2	19

90.220.x010.2	9

90.220.x020.2	14

90.230.0513.2	9

90.230.0514.2	11

90.230.0523.2	14

90.230.0524.2	16

90.230.0526.2	3

90.230.0544.2	21

90.230.1040.2	20

90.230.2014.2	11

90.230.3040.2	20

90.230.x010.2	8

90.230.x020.2	13

90.230.x024.2	17

90.240.0503.2	24

90.240.0504.2	25

90.240.0505.2	25

90.240.0506.2	23

90.240.1000.2	24

90.240.3000.2	24

99.413.0000.8	4

99.415.0000.8	14

99.419.0000.8	3

99.428.0000.8	17

99.450.0000.8	22

99.451.0000.8	26